College of Engineering

School of Aeronautics and Astronautics

AAE 421

Flight Dynamics and Controls

EXAM 2

Author:

Tomoki Koike

Supervisor:

Ran Dai

November 20th, 2020

Purdue University

West Lafayette, Indiana

I certify that I have neither given help to, nor received help from, any individual in matters relating to this examination.

Signature:

Problem 1 (25 pts)

Given the following set of nonlinear differential equations:

V=T-D-mgsinγ/m

γ=L-mgcosγ/mV

h=Vsinγ

Where

L=12ρV2SCL      and       CL=CLαα

D=12ρV2SCD0+ϵCL2

The parameters are set as

ϵ= 0.9,    AR=0.86

m=0.003 kg,  S=0.017 m2

CD0=0.02,  ρ=0.41405 kg/m2

h=10,000 m,  CLα=1.2936

- Find the trim condition for a leveled flight at altitude of h=10,000 m flying at a speed of Ve=V0=3.7 m/s, where the state x=Vγh and the control is denoted by u=αT. (Hint: to maintain a trim condition at level flight, we must have V=γ=h=0 and γ=0)

For the trim conditions we solve the following two equations by plugging in all the given parameters as well as trim conditions.

V=T-D-mgsinγ/m

γ=L-mgcosγ/mV

That gives the following equations

0=-ρS2mcD0+ϵCLα2αe2Ve2-gsinγe+Tem#1

0=ρS2mCLααeVe-gcosγeVe#2

Solving the second equation gives us

αe=0.4722 rad  .

And plugging this into equation (1) gives the trim value of T

Te=0.0171 N  .

Also,

Ve=3.7 m/s

γe=0  rad

he=10,000 m  .
Develop a Simulink model to simulate the system state response for t = 10 sec with the initial condition set as the trim condition in Part (a) and input u=αtrimTtrim×105% .

The Simulink model is the following

The Embedded MATLAB Blocks have the following functions defined

function Vdot = fcn(m, g, T, D, gamma)

Vdot = (T - D - m * g * sin(gamma)) / m;

end

function gammadot = fcn(m, g, L, gamma, V)

gammadot = (L - m * g * cos(gamma)) / m / V;

end

function hdot = fcn(V, gamma)

hdot = V * sin(gamma);

end

The Simulation results is the following

Running the following commands give us the simulation

% Problem 1

% Parameters

epsilon = 0.9;

AR = 0.86;

m = 0.003;

S = 0.017;

cD0 = 0.02;

rho = 0.41405;

h = 10000;

cLa = 1.2936;

g = 9.81;

% (a)

% Equilibrium conditions

he = h;

Ve = 3.7;

gamma = 0;

% Find trim conditions

syms alpha T

cL = cLa * alpha;

L = 0.5*rho*Ve^2*S*cL;

D = 0.5*rho*Ve^2*S*(cD0 + epsilon*cL^2);

eqn1 = 0 == (T - D - m*g*sin(gamma)) / m;

eqn2 = 0 == (L - m*g*cos(gamma)) / m / Ve;

res = solve([eqn1, eqn2], [alpha, T]);

% (b)

alpha_trim = double(res.alpha)

T_trim = double(res.T)

simout = sim("e2_p1_model.slx");

t = simout.tout;

Vsim = simout.vout.signals.values;

gammasim = simout.gammaout.signals.values;

hsim = simout.hout.signals.values;

% Plotting

fig1 = figure('Renderer',"painters", 'Position', [10 10 900 1000]);

subplot(3,1,1)

plot(t,Vsim)

grid on; grid minor; box on;

ylabel('V [m/s]')

subplot(3,1,2)

plot(t,gammasim)

grid on; grid minor; box on;

ylabel('$\gamma$ [rad]')

subplot(3,1,3)

plot(t,hsim)

grid on; grid minor; box on;

ylabel('h [m]')

xlabel('time, [sec]')

sgtitle('Problem 1 (b) $0\leq t \leq 10$ - T. Koike')

saveas(fig1, fullfile(fdir, "p1_2.png"));

- Make use of the MATLAB command ‘linmod.m’ to find the linearized state space model about the trim condition found in part (a), assuming the output y=x.

The following command gives us the A, B, C, and D matrices

[A, B, C, D] = linmod('e2_p1_model',[Ve;gamma;he],[alpha_trim; T_trim])

A=-3.0889-9.810001.43320003.70000,  B=-22.8430333.33335,6150000

C=100010001,  D=000000

Problem 2 (25 pts)

Using Dutch roll approximation, determine the state feedback gains so that the damping ratio and frequency of the Dutch roll are 0.3 and 1.0 rad/s, respectively. Assume the airplane has the following characteristics:

Yβ=-19.5 ft/s2

Yr=1.3 ft/s

Nβ=1.5 s-1

Nr=-0.21 s-1

U0=400 ft/s  .

If

u=-Kx

K=-0.34010.6813

Problem 3 (25 pts)

For the pitch displacement autopilot system shown below

- Determine the gain necessary to improve the system characteristic so that the control system has the following performance: ζ=0.3 and ωn=2.0 rad/s. Verify your solution by providing the root locus plot of the overall system and plot of system response to a 5° step change in the commanded pitch attitude.

Use the Control Systems Designer to tune the gyro gains for the given requirements

num = conv([0, 10], 2.0*[1, 0.3]);

den = conv([1, 10], [1, 0.65, 2.15]);

sys = tf(num, den);

s = tf("s");

H = 1*s;

controlSystemDesigner(sys);

The control system tuner gives us the following gains

Kvg=0.76561

Krg=0.392

The Root locus of the inner loop system becomes

The Root Locus of the outer loop system becomes

Then we get the response for 5-degree pitch attitude change

num = conv([0, 10], 2.0*[1, 0.3]);

den = conv([1, 10], [1, 0.65, 2.15]);

Krg = 0.392;

Kvg = 0.76561;

The response to 5-degree (π/36) change in pitch is

- Replace the rate gyro and amplifier with a PID controller shown below:

Design the PID gains using MATLAB Control System Tuner. Compare the design results with Part (a) by providing the plot of system response to a 5-degree step change in the commanded pitch altitude.

The requirements are

%OS=exp-ζπ1-ζ2×100 =32.23%

timeconstant≔τ=1ζωn=1.6667

The model is as follows.

The results are

Kp+Ki×1s+Kd×sTf+1

Kp=-0.148,  Ki=0.541,  Kd=0.35,  Tf=0.325

The response to a 5-degree step change is as follows.

The PID tuned results seem to be more stable than the one tuned by 2 proportional controllers inside feedback loops.

Problem 4 (25 pts)

Bonus (5 pts)

Use state feedback to design an altitude hold control system. Assume the forward speed is held constant and the longitudinal equation can be modeled using the short-period approximation. The short-period equations are

ΔαΔqΔθ=-1.510-4.0-1.00010ΔαΔ qΔθ+-0.2-8.00Δδe

Assume the Δh=u0Δθ-Δα where u0=200 ft/s. Determine the state feedback gain if the closed-loop eigenvalues are located at

λ1=-1.5±2.5j

λ2=-0.75±1.0j

The state space equation can be remodeled by including the altitude change, which becomes as follows.

ΔαΔqΔθΔh=-1.5100-4.0-1.0000100-20002000ΔαΔqΔθΔh+-0.2-8.000Δδe

First, we have to find out if this system is controllable or not.

% Setup

A = [-1.5, 1, 0, 0; -4, -1, 0, 0; 0, 1, 0, 0; -200, 0, 200, 0];

B = [-0.2; -8; 0; 0];

% Check controllability

Qc = ctrb(A, B);

Qc_rref = rref(Qc);

| Qc = 4×4 103 × -0.0002 -0.0077 0.0204 -0.0085 -0.0080 0.0088 0.0220 -0.1034 0 -0.0080 0.0088 0.0220 0 0.0400 -0.0600 -2.3100 | Qc_rref = 4×4 1 0 0 0 0 1 0 0 0 0 1 0 0 0 0 1 |
| --- | --- |

The controllability matrix has full rank, and therefore, this system is controllable.

Now we find the controller gains using the Brogan’s Algorithm

Step 1:

Find

ϕ=xIn×n-A-1

⇒2 x+12x2+5x+1122x2+5x+1100-82x2+5x+112 x+32x2+5x+1100-8x (2x2+5x+11)2 x+3x (2x2+5x+11)1x0-400 x2+x+4x2(2x2+5x+11)600x2 (2x2+5x+11)200x21x

Step 2:

Compute

ψ=ϕB

⇒-0.4000 x+12x2+5x+11-162x2+5x+111.60002x2+5x+11-8 2 x+32x2+5x+111.6000x(2x2+5x+11)-8 2 x+3x (2x2+5x+11)80 x2+x+4x2 (2x2+5x+11)-4800x2(2x2+5x+11)

Step 3:

Calculate

ψ=ψ1λ1ψ1λ2ψ1λ3ψ1λ4

Where ψ1x,ψ2x correspond to the columns of ψ.

ψ=2.7774-1.3208 j2.7774+1.3208 j-2.3171+0.6642 j-2.3171-0.6642 j3.5019+6.9434 j3.5019-6.9434 j-2.2020-1.8190 j-2.2020+1.8190 j1.4242-2.2553 j1.4242+2.2553 j-0.1072+2.2824 j-0.1072-2.2824 j-7.2129+112.5808 j-7.2129-112.5808 j-5.0239-438.2218 j-5.0239+438.2218 j

Step 4:

Find the gains with

K=-Eψ-1

Where

E=1111

Thus,

K=0.7734-0.2693-1.5781-0.0059

And

Acl=A-BK=-1.34530.9461-0.3156-0.00122.1870-3.1547-12.6248-0.04740100-20002000.

eigAcl=-1.5000-2.5000 i-1.5000+2.5000 i-0.7500-1 i-0.7500+1 i

Thus, our gains are correct.

% Brogan's Algorithm with desired poles

lambda = [-1.5+2.5j, -1.5-2.5j, -0.75+1j, -0.75-1j];

syms x

[n, m] = size(B);

phi = inv(x*eye(n)-A);

psi = phi * B;

psibar = ([subs(psi(:), lambda(1)), subs(psi(:), lambda(2)),...

subs(psi(:), lambda(3)), subs(psi(:), lambda(4))]);

K = -[1, 1, 1, 1]*inv(psibar);

Acl = A - B*K

eig(Acl)