College of Engineering

School of Aeronautics and Astronautics

AAE 532

Orbital Mechanics

PS 6

In-Plane Maneuver

Author:

Tomoki Koike

Supervisor:

K. C. Howell

October 16th, 2020 Friday

Purdue University

West Lafayette, Indiana

Problem 1: Assume a relative two-body model and a space vehicle that is currently in a Mars orbit with rp=1.1R♂ and ra=6.0R♂. The spacecraft is currently located at θc*=90° at time tc. A single in-plane adjustment will be employed to circularize the orbit.

- At what true anomaly values does r=4.5R♂? (Note that two locations exist!) Select the location that is the earliest opportunity after tc to reach r=4.5R♂. let this time be t1. Determine v1,γ1,E1,t1-tp at this location.

From the radius of periapsis and apoapsis we can find the orbital parameters to define the orbit.

a=0.5rp+ra=0.5(1.1R♂+6.0R♂)=4R♂  .

Since, R♂= 3397 km, the semi-major axis becomes

a = 1.2059e+4  km  .

Next, from the periapsis and apoapsis we can calculate the eccentricity

e=ra-rpra+rp= 0.6901  .

Then, the semi-latus rectum, p becomes

p=a1-e2= 6.3155e+3 km  .

When the gravitational parameter, μ≅42828  km3/s2, the specific angular momentum becomes

h=μp=1.6446e+4  km2s  .

Then, at r=4.5R♂ we can find the true anomaly by

θ1*=arccos1ep4.5R♂-1

θ1*=±148.2486°

The earliest to θc*=90° is the positive value. Thus,

θ1*=148.2486°  .

The velocity at this point is

v1=μ2r1-1a = 1.4325  km/s  .

The flight path angle is positive because at time t1 it is ascending,

γ1=arccoshr1v1= 41.317°   .

Using the equation below the eccentric anomaly becomes,

tanθ*2=1+e1-e1/2tanE2

E1=112.81°  .

Using, the equation on notes G7 we can find the elapsed time as the following

μa3t1-tp=E1-esinE1

t1-tc=a3μ(E1-esinE1)=7.1785e+5  s = 8.3084 days  .

We check if this is an elliptical orbit

v1=1.4325  km/s<2.3672  km/s=2μr1=2vc.

- Sketch the orbit. Mark the usual quantities at the time t1:r1,v1,γ1,l.h.,E1; also add appropriate unit vectors e,p,r1,θ1.

l.h

v1

γ1

θ1

r1

r1

E1

p

e

θ1*

- What is the “wait time” till the maneuver t1-tc?

Similar to E1, we can find Ec using θc*.

Ec=46.359°  .

Then, like in the previous part (a), where we found the time difference for point 1, we can find,

tc-tp=a3μEc-esinEc= 2.9346e+5  s = 3.3965 days   .

Thus, the wait time becomes

t1-tp-tc-tp=t1-tc=4.2439e+5  s= 4.9119  days  .

- Determine r1+,v1+,γ1+ after the maneuver. Compute the required maneuver Δv,α. Recall that Δv=Δv. [Include VECTOR diagrams!!!!]

Since after the maneuver the spacecraft is going to enter a circular orbit, we know that the radius of this circle is going to be equivalent with the distance at the point at time t1. Hence,

r1+=r1=4.5R♂=1.5287e+4  km  .

And the velocity after the maneuver is going to be equal to the circular velocity,

v1+=vc=μr1 =1.6738   km/s  .

Now, since the new velocity is tangent to a circle the direction of it is parallel to θ1, its vector form is

v1+=v1+θ1  .

And because v1 can be expressed as

v1=v1sinγ1r1+cosγ1θ1  .

Thus, Δv becomes

Δv = v1+θ1-v1sinγ1r1+cosγ1θ1=-v1sinγ1r1+(v1+-v1cosγ1)θ1

Δv=-0.94575r1+1.6738-1.0759θ1=(-0.94575r1+0.59795θ1)  km/s  .

Thus,

Δv = 1.1189  km/s  .

Then the angle α can be found by the dot product rule and geometry (negative sign since it is moving away from the center of orbit),

α=arccosv1+∙Δv v1+Δv +γ1

α=-99.014°  .

The angle alpha is equal to the flight path angle since the flight path angle is the angle between θ1 and v1.

l.h

Δv

v1+

α

- Plot the old and new orbits on the same figure using your MATLAB script. On the plot, mark r0,r1,v1-,l.h.,γ1-,v1+,γ1+,Δv,α.

α

l.h

r1

γ1+=0

v1+

Δv

v1-

γ1-

r0

- Plot the two orbits in GMAT using Mars as the central body. At the maneuver time, use a report to list r,v in each orbit at the maneuver time. Choose a convenient set of unit vectors (coordinate frame). Subtracting the velocity vectors should yield your Δv=Δv. Does it?

From the GMAT report we have the following results for the position and velocity vectors at the maneuver point for the two orbits. The raw data is the following

Elliptical orbit:

|  | e | p |
| --- | --- | --- |
| r | -1.3e+4 km | 8044 km |
| v | -1.37 km/s | -0.4173 km/s |

Circular orbit:

|  | e | p |
| --- | --- | --- |
| r | -1.3e+4 km | 8044 km |
| v | -0.8808 km/s | -1.4230 km/s |

Thus, the velocity difference from the GMAT results become

Δv=-0.8808+1.37e+-1.423+0.4173p   km/s

Δv=0.4892e-1.0057p   km/s  .

Now since

r=cosθ*e+sinθ*p

θ=cosθ*+π2e+sinθ*+π2p=-sinθ*e+cosθ*p

Δvorbital=Δvcosθ*-sinθ*sinθ*cosθ*

Δvorbital=(0.4892-1.0057)cosθ*-sinθ*sinθ*cosθ*

where θ*=148.2486°, therefore

Δvorbital=(-0.94575r1+0.59795θ1)  km/s   .

And,

Δvorbital=1.1189km/s  .

This agrees with our result in part (d). Thus, we have verified our result.

Problem 2: given a two-body model, vehicle is successfully launched into an Earth orbit with e=0.4 and a=4R⊕. A single in-plane maneuver will be implemented when θ*=135°. Let the maneuver be defined as Δv=0.90 km/s,  α=+45°.

- Determine r.v-,γ- at the moment of maneuver point.

Firstly, the semi latus rectum is

p =a1-e2= 21431  km  .

Then, we can find the magnitude of r

r=p1+ecosθ* = 29883  km  .

Since, μ=398600.4415  km3/s2, the magnitude of the velocity vector is

v=μ2r-1a = 3.3248  km/s  .

The specific angular momentum is

h=μp=92424  km2/s  .

The flight path angle is

γ-=arccoshrv=21.524°    ∵θ*<180°.

The position vector can be expressed by its magnitude and true anomaly

r=rcosθ*e+sinθ*p=-21130e+21130p  km

or

r=29883r  km

Then the velocity vector becomes

v=vsinγ r+cosγθ=1.2198r +3.0929θ  km/s

or in the e-p frame

v=vorbitalcosθ*sinθ*-sinθ*cosθ*=-3.0496e-1.3245p  km/s

- Express the maneuver in both r,θ and  e,p unit vectors. Also determine the maneuver in the VNB set of coordinates.

- Prepare any VECTOR diagrams!!!! Determine r+,v+,γ+ in the new orbit immediately after the maneuver.

l.h

The vector diagram at the maneuver point is

r

B

v+

Δγ

Δv

v-

θ

α

From the cosine law

vN2=(Δv)2+v2-2(Δv)vcos180°-45°

vN=v+=4.012  km/s

The corresponding flight path angle is

γ+=arccoshrvN=39.563°

Δγ=γ+-γ-=18.039°

The vector of vN becomes,

v+=v+sinγ+r+cosγ+θ=2.5553r+3.0929θ  km\s  .

Thus, the velocity vector of the difference of the velocity vectors become

Δv=v+-v-=2.5553r+3.0929θ km\s-1.2198r+3.0929θ km/s

Δv=1.3355r   km/s

In e,p unit vectors the two become

v+=2.55533.0929cosθ*sinθ*-sinθ*cosθ*=-3.9939e-0.38013p  km/s

Δv=1.33550cosθ*sinθ*-sinθ*cosθ*=-0.94434e+0.94434p  km/s

In the VNB coordinate system,

V=v-

and B is normal to V on the same plane depicted in the vector diagram on the previous page.

v-=3.3248V  km/s

v+=v+cosΔγV+sinΔγB=3.8147V+1.2424B  km/s  .

Δv=v+-v-=0.48998V+1.2424B  km/s  .

And finally,

r+=r-=29883km.

- To determine the impact that such a maneuver creates on the orbital characteristics, compute the following characteristics of the new orbit: a,e,h,P,E,θ*,E,γ,t-tp,rp,Δω.

Now, the semi major axis is

a=-μ2v+22-μr+=37668  km  .

The eccentricity becomes from notes JS3

e=r+v+2μ-12cos2γ++sin2γ+

e = 0.65656  .

Then the semi latus rectum becomes

p=a1-e2=21431  km  .

Then the specific angular momentum becomes

h=μp=92424  km2/s  .

Then the period becomes

P=2πa3μ = 72756 s = 0.84208 days  .

The specific energy becomes

E=-μ2a=-5.291 km2/s2  .

We know from the previous problem that the flight path angle is

γ=γ+=39.563°  .

The true anomaly is positive since the flight path angle is positive, and therefore,

θ*=+arccos1epr+-1=115.52°   .

The eccentric anomaly is computed by

tanθ*2=1+e1-e1/2tanE2

E = 71.651°  .

Then, the time elapsed from periapsis is

t-tp=E-sinEa3μ=3490 s  .

The radius of periapsis is

rp=a1-e= 12937  km  .

Finally,

Δω= 135°-115.52°= 19.482°   .

- Plot the new and the old orbits in Matlab on the same figure using your Matlab script. On the plot, mark r0,r1,v1-,l.h.,γ1-,v1+,γ1+,Δv,α. Also indicate the new and old lines of apsides and the shift, i.e., Δω. Is it positive or negative? Why?

r0

l.h.

v1-

γ1+

γ1-

v1+

Δv

α

New line of apsides

Δω

Old line of apsides

r1

Δω is a positive value because this value is the difference between the true anomaly of the new orbit and the old orbit, and that calculation yields a positive value. Also, this value indicates the angle in which the line of apsides rotates and from the old orbit to the new one the line of apsides rotates in a positive angle with respect to the h vector which is coming out of the page.

- Bonus  You can use GMAT in two ways to check the maneuver and the new orbit. Use either method to check your results:

Use a start date October 10, 2020 12:00:00 to propagate the satellites. Put in the old and new orbits with 2 satellites and compare the velocities at the intersection point to assess if the difference equals the required Δv;

Sat1 in the figure orbits the old orbit and Sat2 orbits the new one.

The summary of Sat1 and Sat2 at the maneuver point is the following

Propagate Command: Propagate1

Spacecraft       : Sat1

Coordinate System: EarthMJ2000Eq

Time System   Gregorian                     Modified Julian

----------------------------------------------------------------------

UTC Epoch:    10 Oct 2020 23:15:55.660      29133.4693942075

TAI Epoch:    10 Oct 2020 23:16:32.660      29133.4698224483

TT  Epoch:    10 Oct 2020 23:17:04.844      29133.4701949483

TDB Epoch:    10 Oct 2020 23:17:04.842      29133.4701949292

Cartesian State                       Keplerian State

---------------------------           --------------------------------

X  =  -21130.064944958 km             SMA  =   25513.000000033 km

Y  =   21130.802534376 km             ECC  =   0.4000000000002

Z  =   0.0000000000000 km             INC  =   0.0000000000000 deg

VX =  -3.0495872416288 km/sec         RAAN =   0.0000000000000 deg

VY =  -1.3244038463119 km/sec         AOP  =   360.00000000000 deg

VZ =   0.0000000000000 km/sec         TA   =   134.99900000255 deg

MA   =   94.643047549128 deg

EA   =   115.35387465803 deg

Spherical State                       Other Orbit Data

---------------------------           --------------------------------

RMAG =   29882.945978014 km           Mean Motion        =   1.549268155e-04 deg/sec

RA   =   134.99900000253 deg          Orbit Energy       =  -7.8117124897009 km^2/s^2

DEC  =   0.0000000000000 deg          C3                 =  -15.623424979402 km^2/s^2

VMAG =   3.3247598247740 km/s         Semilatus Rectum   =   21430.920000024 km

AZI  =   90.000000000000 deg          Angular Momentum   =   92424.965100133 km^2/s

VFPA =   68.475802105204 deg          Beta Angle         =  -6.9917202653511 deg

RAV  =  -156.52519789227 deg          Periapsis Altitude =   8929.6637000150 km

DECV =   0.0000000000000 deg          VelPeriapsis       =   6.0377693136860 km/s

VelApoapsis        =   2.5876154201499 km/s

Orbit Period       =   40555.828160079 s

Planetodetic Properties

---------------------------

LST       =   135.26514928876 deg

MHA       =   8.9964759666473 deg

Latitude  =  -0.0802677349499 deg

Longitude =   126.26867332211 deg

Altitude  =   23504.809719853 km

Spacecraft Properties

------------------------------

Cd                    =   2.200000

Drag area             =   15.00000 m^2

Cr                    =   1.800000

Reflective (SRP) area =   1.000000 m^2

Dry mass              =   850.00000000000 kg

Total mass            =   850.00000000000 kg

SPADDragScaleFactor   =   1.000000

SPADSRPScaleFactor    =   1.000000

========================================================================

Spacecraft       : Sat2

Coordinate System: EarthMJ2000Eq

Time System   Gregorian                     Modified Julian

----------------------------------------------------------------------

UTC Epoch:    10 Oct 2020 12:00:00.000      29133.0000000000

TAI Epoch:    10 Oct 2020 12:00:37.000      29133.0004282407

TT  Epoch:    10 Oct 2020 12:01:09.184      29133.0008007407

TDB Epoch:    10 Oct 2020 12:01:09.182      29133.0008007216

Cartesian State                       Keplerian State

---------------------------           --------------------------------

X  =  -21131.423961286 km             SMA  =   37668.000000000 km

Y  =   21129.948761085 km             ECC  =   0.6565600000000

Z  =   0.0000000000000 km             INC  =   0.0000000000000 deg

VX =  -3.9938238313617 km/sec         RAAN =   0.0000000000000 deg

VY =  -0.3802198116453 km/sec         AOP  =   19.482000000000 deg

VZ =   0.0000000000000 km/sec         TA   =   115.52000000000 deg

MA   =   35.947110547546 deg

EA   =   71.653010021515 deg

Spherical State                       Other Orbit Data

---------------------------           --------------------------------

RMAG =   29883.303252446 km           Mean Motion        =   8.635948526e-05 deg/sec

RA   =   135.00200000000 deg          Orbit Energy       =  -5.2909690121589 km^2/s^2

DEC  =   0.0000000000000 deg          C3                 =  -10.581938024318 km^2/s^2

VMAG =   4.0118818403737 km/s         Semilatus Rectum   =   21430.416306355 km

AZI  =   90.000000000000 deg          Angular Momentum   =   92423.878955830 km^2/s

VFPA =   50.436279597976 deg          Beta Angle         =  -6.8144880605859 deg

RAV  =  -174.56172040202 deg          Periapsis Altitude =   6558.5616200000 km

DECV =   0.0000000000000 deg          VelPeriapsis       =   7.1443176247429 km/s

VelApoapsis        =   1.4811684726431 km/s

Orbit Period       =   72756.169034291 s

Planetodetic Properties

---------------------------

LST       =   135.26813282482 deg

MHA       =   199.55190411210 deg

Latitude  =  -0.0804990060739 deg

Longitude =  -64.283771287277 deg

Altitude  =   23505.166994526 km

Spacecraft Properties

------------------------------

Cd                    =   2.200000

Drag area             =   15.00000 m^2

Cr                    =   1.800000

Reflective (SRP) area =   1.000000 m^2

Dry mass              =   850.00000000000 kg

Total mass            =   850.00000000000 kg

SPADDragScaleFactor   =   1.000000

SPADSRPScaleFactor    =   1.000000

The red bolded values are the velocities for Sat1 and Sat2 at the maneuver point.

For Sat1

| Vx | -3.0495872416288 km/sec |
| --- | --- |
| Vy | -1.3244038463119 km/sec |
| Vz | 0.0000000000000 km/sec |

For Sat2

| Vx | -3.9938238313617 km/sec |
| --- | --- |
| Vy | -0.3802198116453 km/sec |
| Vz | 0.0000000000000 km/sec |

Then the difference of these two vectors become

ΔV=-0.9442365897e+0.9441840347p

Which is approximately

ΔV=-0.9443e+0.9443p

This agrees with out results on page 12. Thus, we have verified our results using GMAT.

Problem 3: Assume a relative two-body model and a space vehicle that is currently located in an Earth orbit with a = 3R⊕ and e =0.6. A single in-plane adjustment is to be implemented for a perigee-raise maneuver. The goal is a new periapsis distance such the new rp=2R⊕. At the same time, it is desired to produce an orbit that is less eccentric, i.e., the new eccentricity is e=0.4 and both goals are accomplished using the same maneuver. The maneuver will take place when the spacecraft is located at the end of the minor axis and descending.

- Determine r1-,v1-,γ1-,t1-tp at the maneuver point. Sketch or plot the orbit in MATLAB. Mark the usual quantities at the maneuver location prior to the maneuver: r-,v-,γ-,l.h.,E-; also add appropriate unit vectors e,p;r,θ.

Plugging in the radius of the Earth we have

a = 19134  km

And we are given that

E-=-90°=270°

Using the following relationship

tanθ*2=1+e1-e1/2tanE2

We find that the true anomaly is

θ*=-126.87°=233.13°

the semi latus rectum is

p =a1-e2= 12246  km  .

Then, we can find the magnitude of r

r1-=p1+ecosθ*=19134  km.

Since, μ=398600.4415  km3/s2, the magnitude of the velocity vector is

v1-=μ2r-1a=4.5642  km/s.

The specific angular momentum is

h=μp=69866  km2/s  .

The flight path angle is

γ1-=arccoshr1-v1-=-36.870°       ∵θ*>180°  .

The mean anomaly becomes

M = E – esinE

M = -32.704° = 327.30°

The time elapse from periapsis is

t1-tp=Ma3μ = 23948  s = 0.27718 days

Using the radial distance, we can express the r vector in the orbital frame

r1-=r1-r=19134  km  r  .

In the other frame this is

r1-=r1-cosθ*e+sinθ*p

r1-=-11481e-15308p   km  .

Then, the velocity vector becomes

v-=v1-sinγ1-r+cosγ1-θ

v-=-2.7385r+3.6513θ  km/s  .

The velocity vector is also represented as

v-=-2.7385r+3.6513θcosθ*sinθ*-sinθ*cosθ*=4.5642e  km/s  .

The orbit is plotted on the next page

E-

e

p

θ

l.h

r

r-

v-

γ-

θ*

- Determine r+,v+,γ+ at the maneuver point. [Include the vector diagrams!!!!] Compute the required maneuver (Δv, α). Express that maneuver in terms of r,θ,V,B sets of unit vectors.

From the requirements of the new orbit we can compute the new semi major axis

a+=rp+1-e+=2R⊕1-0.4=12756  km  .

Then, the new semi latus rectum becomes

p+=a+1-e+2=17859  km  .

Then distance from Earth is the same as r-

r+=r-=19134  km  .

Thus, we can get the true anomaly for the new orbit

θ*+=±arccos1epr+-1=±99.594°

The new velocity becomes

v+=μ2r+-1a+=4.7869  km/s  .

Then the new flight path angle is going to become

γ+=arccosμp+r+v+=±22.908°

If γ+=22.908°,

tanθ*+=r+v+2μcosγ+sinγ+r+v+2μcos2γ±1

θ*+=arccosr+v+2μcosγ+sinγ+r+v+2μcos2γ±1=-80.406°  or 99.594°

Similarly, if γ+=-22.908°

θ*+=80.406°or-99.594°

If we choose a positive flight path angle,

r-=r+

v+

l.h

r

γ-

v-

γ+

α

θ

Δv

B

V

v+

Δv

α

r

γ-

γ+

θ

l.h

r-=r+

v-

If we choose a negative flight path angle,

We want to choose a Δv that is smaller so we define

γ+=-22.908°

θ*+=-99.594°

Thus, the new orbit is descending as well. Hence the position and velocity vectors will become  r+,v+,γ+

r+=r+r=19134  km  r  .

v+=v+sinγ+r+cosγ+θ

v+=-1.8633r+4.4094θ  km/s  .

The maneuver becomes

Δv=v+-v-=0.87519r+0.75807θ  km/s  .

Δv=1.1579  km/s  .

and the angle becomes

α=arccosv-∙Δvv-Δv=85.971°

The angle alpha is negative since the Earth is oriented above the local horizon and the Δv is moving downwards (moving away from the Earth).

In the VNB coordinate system, we can define the velocity change as

Δv=ΔvcosαV+sinαB=0.081344V+1.1550B  km/s  .

- Determine the characteristics of the new orbit: a,e,rp,ra,P,E;θ*,E,γ,t-tp,Δω. How long till the spacecraft reaches perigee in the new orbit?

From the calculations of the previous parts of this problem we know that

a+=12756  km

e+=0.4

θ*+=-99.594°

γ+=-22.908°

The radius of perigee and apogee become

rp+=a1-e+=12756  km

ra+=a1+e+=29765  km  .

The period is

P=2πa+3μ = 30851 s = 0.35707 days  .

The specific energy is

E=-μ2a+=  -9.3742 km2/s2  .

From the relationship

tanθ*2=1+e1-e1/2tanE2

the eccentric anomaly becomes,

E+=-75.522°  .

The elapsed time from the perigee is

t-tp=E+-sinE+a+3μ= 29133  s = 0.33719 days  .

Finally,

Δω=θ*--θ*+=-126.87°--99.594°=-27.276°

- Plot the old and new orbits on the same figure using your MATLAB script. On the plot, mark r-,r+,v-,l.h.,γ-,v+,γ+,Δv,α.

Δω

v+

Δv

r-,r+

l.h.

v-

α

γ-

γ+

Appendix

MATLAB Code

Problem 1

% AAE 532 HW 6 Problem 1

% Tomoki Koike

close all; clear all; clc;

fdir = 'C:\Users\Tomo\Desktop\studies\2020-Fall\AAE532\matlab\outputs\ps6';

set(groot, 'defaulttextinterpreter','latex');

set(groot, 'defaultAxesTickLabelInterpreter','latex');

set(groot, 'defaultLegendInterpreter','latex');

format shortG;

% Set constants

planet_consts = setup_planetary_constants();  % Function that sets up all the constants in the table

sun = planet_consts.sun;  % structure of sun

earth = planet_consts.earth;  % structure of earth

mars = planet_consts.mars;  % structure of mars

G = 6.6743015e-20;  % Gravitational constant [km^3/kg/s^2]

% (a)

Rm = mars.mer;

mu = mars.gp;

rp = 1.1 * Rm;  % radius of periapsis

ra = 6.0 * Rm;  % radius of apoapsis

a = 0.5 * (rp + ra);  % semi major axis

e = (ra - rp) / (ra + rp);  % eccentricity

p = a * (1 - e^2);  % semi laturs rectum

h = sqrt(mu * p);  % specific angular momentum

% At time t1

TA_c = 90;  % at time tc

r_1 = 4.5 * Rm;

TA_1 = acos_dbval(1/e * (p/r_1 - 1), "deg");

TA_1 = TA_1(find(min(TA_1 - TA_c)));

v_1 = vis_viva(r_1, a, mu);

FPA_1 = acos_dbval(h / r_1 / v_1, "deg");

FPA_1 = FPA_1(FPA_1 > 0);

EA_1 = T2E_anomaly(e, TA_1, "deg");

EA_c = T2E_anomaly(e, TA_c, "deg");

dt_p1 = sqrt(a^3 / mu) * (EA_1 - e * sind(EA_1));

dt_p1_day = dt_p1 / 60 / 60 / 24;

% (b)

% Ellipse

theta = 0:0.01:2*pi;

R = p ./ (1 + e * cos(theta));

X = R .* cos(theta); Y = R .* sin(theta);

% Mars

Xm = Rm * cos(theta); Ym = Rm * sin(theta);

% At time t1

X1 = r_1 * cosd(TA_1); Y1 = r_1 * sind(TA_1);

% Plot

fig = figure("Renderer","painters","Position",[10 10 900 700]);

plot(X, Y, '-b')

title('Elliptical Orbit of $e=0.6901$ Around Mars - T. Koike')

hold on; grid on; grid minor; box on; axis equal;

plot(X1, Y1, '.r', 'MarkerSize', 18)

% Mars

plot(Xm, Ym, '-m')

plot(0, 0, '.k', 'MarkerSize', 15)

% Axes

nanikore = -ra:rp; korenani = -a*sqrt(1-e^2):a*sqrt(1-e^2);

plot(nanikore, zeros(size(nanikore)), '--k')

plot(-a*e*ones(size(korenani)), korenani, '--k')

hold off

xlabel('$\hat{e}$ [km]')

ylabel('$\hat{p}$ [km]')

saveas(fig, fullfile(fdir, 'p1_orbit1.png'));

% (c)

EA_c = T2E_anomaly(e, TA_c, "deg");

dt_pc = sqrt(a^3 / mu) * (EA_c - e * sind(EA_c));

dt_pc_day = dt_pc / 60 / 60 / 24;

dt_c1 = dt_p1 - dt_pc;

dt_c1_day = dt_c1 / 60 / 60 / 24;

% (d)

r_new = r_1;

v_new = sqrt(mu / r_1);

v_1_vec = v_1 * [sind(FPA_1), cosd(FPA_1)];

v_new_vec = v_new * [0, 1];

dv_vec = v_new_vec - v_1_vec;

dv = norm(dv_vec);

syms alpha

eqn = v_new / sind(180-alpha) == dv / sind(FPA_1);

alpha = solve(eqn, alpha)

temp_ang = acosd(dot(v_new_vec, dv_vec)/ dv / v_new) + FPA_1

% alpha = acosd(dot(v_new_vec, v_1_vec) / v_new / v_1);

% (e)

% Ellipse

theta = 0:0.01:2*pi;

R = p ./ (1 + e * cos(theta));

X = R .* cos(theta); Y = R .* sin(theta);

% Mars

Xm = Rm * cos(theta); Ym = Rm * sin(theta);

% At time t1

X1 = r_1 * cosd(TA_1); Y1 = r_1 * sind(TA_1);

% New circular orbit

Xnew = r_1 * cos(theta); Ynew = r_1 * sin(theta);

% Plot

fig = figure("Renderer","painters","Position",[10 10 900 700]);

plot(X, Y, '-b')

title('Elliptical Orbit of $e=0.6901$ Around Mars with Maneuver to Circular Orbit - T. Koike')

hold on; grid on; grid minor; box on; axis equal;

plot(X1, Y1, '.r', 'MarkerSize', 18)

% Mars

plot(Xm, Ym, '-m')

plot(0, 0, '.k', 'MarkerSize', 15)

% Axes

nanikore = -ra:rp; korenani = -a*sqrt(1-e^2):a*sqrt(1-e^2);

plot(nanikore, zeros(size(nanikore)), '--k')

plot(-a*e*ones(size(korenani)), korenani, '--k')

% New orbit

plot(Xnew, Ynew, '-g')

hold off

xlabel('$\hat{e}$ [km]')

ylabel('$\hat{p}$ [km]')

saveas(fig, fullfile(fdir, 'p1_orbit2.png'));

% (f)

vvec1 = [-1.37, -0.4173];

vvec2 = [-0.8808, -1.4230];

Dv = vvec2 - vvec1;

Dv_orbital = Dv * [cosd(TA_1), -sind(TA_1); sind(TA_1), cosd(TA_1)];

Problem 2

% AAE 532 HW 6 Problem 2

% Tomoki Koike

close all; clear all; clc;

fdir = 'C:\Users\Tomo\Desktop\studies\2020-Fall\AAE532\matlab\outputs\ps6';

set(groot, 'defaulttextinterpreter','latex');

set(groot, 'defaultAxesTickLabelInterpreter','latex');

set(groot, 'defaultLegendInterpreter','latex');

format shortG;

% Set constants

planet_consts = setup_planetary_constants();  % Function that sets up all the constants in the table

sun = planet_consts.sun;  % structure of sun

earth = planet_consts.earth;  % structure of earth

mars = planet_consts.mars;  % structure of mars

G = 6.6743015e-20;  % Gravitational constant [km^3/kg/s^2]

% (a)

Re         = earth.mer;  % Earth radius

mu         = earth.gp;  % gravitational parameter

a          = 4*Re;  % semi major axis

e          = 0.4;  % eccentricity

p          = a * (1 - e^2);  % semi latus rectum

TA         = 135;  % true anomaly at maneuver point

r1         = p / (1 + e * cosd(TA));

v1         = vis_viva(r1, a, mu);

h          = sqrt(mu * p);  % specific angular momentum

FPA1       = acosd(h / r1 / v1);  % flight path angle

r1vec_i     = r1 * [cosd(TA), sind(TA)];

v1vec_o    = v1 * [sind(FPA1), cosd(FPA1)]

v1vec_i    = v1vec_o * [cosd(TA), sind(TA); -sind(TA), cosd(TA)]

% Maneuver

dv = 0.90;  % km/s

alpha = 45;  % degrees

v2 = sqrt(dv^2 + v1^2 - 2*dv*v1*cosd(180 - alpha));

FPA2 = acosd(h / r1 / v2)

dFPA = FPA2 - FPA1

v2vec_o = v2 * [sind(FPA2), cosd(FPA2)]

dvvec_o = v2vec_o - v1vec_o

v2vec_i = v2vec_o * [cosd(TA), sind(TA); -sind(TA), cosd(TA)]

dvvec_i = dvvec_o * [cosd(TA), sind(TA); -sind(TA), cosd(TA)]

v1vec_vbn = v1 * [1, 0];

v2vec_vbn = v2 * [cosd(dFPA), sind(dFPA)];

dvvec_vbn = v2vec_vbn - v1vec_vbn;

% (d)

r2 = r1;

a_new = (-mu / 2) / (v2^2/2 - mu/r2);

e_new = calc_e_posVel(r2, v2, FPA2, mu, "deg")

p_new = a_new*(1 - e_new^2)

h = sqrt(mu * p_new)

IP = 2*pi*sqrt(a_new^3 / mu)

IP_day = IP / 60 /60 / 24

En = -mu / 2 / a_new

TA_new = acos_dbval(1/e_new *(p_new/r2 - 1), "deg")

TA_new = TA_new(TA_new >0)

E_new = T2E_anomaly(e_new, TA_new, "deg")

dt = (deg2rad(E_new) - sind(E_new)) * sqrt(a_new^3 / mu)

rp_new = a_new*(1 - e_new)

Domega = TA - TA_new

ra_new = a_new*(1 + e_new)

rp = a*(1-e);

ra = a*(1+e);

% Plotting for visualization

% old orbit

angles = 0:0.01:2*pi;

RR = p ./ (1 + e*cos(angles)); XX = RR.*cos(angles); YY = RR.*sin(angles);

% new orbit

RR_new = p_new ./ (1 + e_new*cos(angles - deg2rad(Domega)));

XX_new = RR_new.*cos(angles);

YY_new = RR_new.*sin(angles);

rp_vec = rp_new*[cosd(Domega), sind(Domega)];

ra_vec = ra_new*[cosd(Domega+180), sind(Domega+180)];

Xearth = Re*cos(angles); Yearth = Re*sin(angles);

fig = figure("Renderer","painters","Position",[10, 10, 900, 700]);

plot(XX,YY)

hold on; grid on; grid minor; box on; axis equal;

plot(r1vec_i(1), r1vec_i(2), '.r', 'MarkerSize', 20)

plot(0,0,'.k', 'MarkerSize', 15)

plot(Xearth, Yearth, '-g')

plot(XX_new, YY_new, '-m')

plot([-ra, rp], [0, 0], '--k')

plot([ra_vec(1), rp_vec(1)],[ra_vec(2), rp_vec(2)], '--r')

hold off

title('PS6 P2 Maneuver Example Orbit - T. Koike')

xlabel('$\hat{e}$ [km]')

ylabel('$\hat{p}$ [km]')

saveas(fig, fullfile(fdir, "p2-orbit.png"))

function e = calc_e_posVel(r, v, gamma, mu, unit)

A = (r*v^2 / mu - 1)^2;

if unit == "rad"

B = cos(gamma)^2;

C = sin(gamma)^2;

else

B = cosd(gamma)^2;

C = sind(gamma)^2;

end

e = sqrt(A*B+C);

end

Problem 3

% AAE 532 HW 6 Problem 2

% Tomoki Koike

close all; clear all; clc;

fdir = 'C:\Users\Tomo\Desktop\studies\2020-Fall\AAE532\matlab\outputs\ps6';

set(groot, 'defaulttextinterpreter','latex');

set(groot, 'defaultAxesTickLabelInterpreter','latex');

set(groot, 'defaultLegendInterpreter','latex');

format shortG;

% Set constants

planet_consts = setup_planetary_constants();  % Function that sets up all the constants in the table

sun = planet_consts.sun;  % structure of sun

earth = planet_consts.earth;  % structure of earth

mars = planet_consts.mars;  % structure of mars

G = 6.6743015e-20;  % Gravitational constant [km^3/kg/s^2]

% (a)

Re         = earth.mer;  % Earth radius

mu         = earth.gp;  % gravitational parameter

a          = 3*Re;  % semi major axis

e          = 0.6;  % eccentricity

p          = a * (1 - e^2);  % semi latus rectum

E          = -90;  % true anomaly at maneuver point

M          = rad2deg(deg2rad(E) - sind(E))

TA         = E2T_anomaly(e, E, "deg")

r1         = p / (1 + e * cosd(TA));

v1         = vis_viva(r1, a, mu);

h          = sqrt(mu * p);  % specific angular momentum

FPA1       = -acosd(h / r1 / v1)  % flight path angle

dt_1p      = deg2rad(360+M)*sqrt(a^3 / mu)

dt_1p_day  = dt_1p / 60 / 60 / 24

ra         = a*(1+e); rp = a*(1-e); b = a*sqrt(1-e^2);

r1vec_o    = r1 * [1, 0];

r1vec_i    = r1 * [cosd(TA), sind(TA)];

v1vec_o    = v1 * [sind(FPA1), cosd(FPA1)];

v1vec_i    = v1vec_o * [cosd(TA), sind(TA); -sind(TA), cosd(TA)];

% Plotting for visualization

% old orbit

angles = 0:0.01:2*pi;

RR = p ./ (1 + e*cos(angles)); XX = RR.*cos(angles); YY = RR.*sin(angles);

% Reference circle

Xref = a*cos(angles)-a*e; Yref = a*sin(angles);

% new orbit

% RR_new = p_new ./ (1 + e_new*cos(angles - deg2rad(Domega)));

% XX_new = RR_new.*cos(angles);

% YY_new = RR_new.*sin(angles);

% rp_vec = rp_new*[cosd(Domega), sind(Domega)];

% ra_vec = ra_new*[cosd(Domega+180), sind(Domega+180)];

% Earth

Xearth = Re*cos(angles); Yearth = Re*sin(angles);

fig = figure("Renderer","painters","Position",[10, 10, 900, 700]);

plot(XX,YY)

hold on; grid on; grid minor; box on; axis equal;

% Maneuver point

plot(r1vec_i(1), r1vec_i(2), '.r', 'MarkerSize', 20)

% Center of orbit

plot(0,0,'.k', 'MarkerSize', 15)

% Reference circle

plot(Xref, Yref, '-m')

% Semi minor axis

plot([-a*e, -a*e], [-b, b], '--k')

% Earth

plot(Xearth, Yearth, '-g')

% plot(XX_new, YY_new, '-m')

plot([-ra, rp], [0, 0], '--k')

% plot([ra_vec(1), rp_vec(1)],[ra_vec(2), rp_vec(2)], '--r')

hold off

title('PS6 P3 Orbit without Maneuver - T. Koike')

xlabel('$\hat{e}$ [km]')

ylabel('$\hat{p}$ [km]')

saveas(fig, fullfile(fdir, "p3-orbit-noManeuver.png"))

% New orbit after maneuver

rp2 = 2*Re

e2 = 0.4

a2 = rp2 / (1 - e2)

p2 = a2 * (1 - e2^2)

r2 = r1

TA2 = acos_dbval(1/e2 * (p2/r2 - 1), "deg")

v2 = vis_viva(r2, a2, mu)

FPA2 = acos_dbval(sqrt(mu*p2)/r2/v2, "deg")

TA2_eval1 = TAfromRVGamma(r2, v2, FPA2(1), mu, "deg")

TA2_eval2 = TAfromRVGamma(r2, v2, FPA2(2), mu, "deg")

FPA2 = FPA2(FPA2 < 0)

TA2 = TA2(TA2 < 0)

r2vec_o = r2 * [1, 0];

v2vec_o = v2 * [sind(FPA2), cosd(FPA2)]

dvvec_o = v2vec_o - v1vec_o

dv = norm(dvvec_o)

alpha = acos_dbval(dot(v1vec_o, dvvec_o)/v1/norm(dvvec_o), "deg")

alpha = alpha(alpha > 0)

dvvec_VBN = dv * [cosd(alpha), sind(alpha)]

rp2 = a2*(1-e2)

ra2 = a2*(1+e2)

IP = 2*pi*sqrt(a2^3/mu)

IP_day = IP / 60 / 60 / 24

En = -mu/2/a2

E2 = T2E_anomaly(e2, TA2, "deg")

dt_2p = (deg2rad(360 + E2) - sind(360 + E2))*sqrt(a2^3/mu)

dt_2p_day = dt_2p / 60/60/24

Domega = (TA-360) - TA2

% Plotting for visualization

% old orbit

angles = 0:0.01:2*pi;

RR = p ./ (1 + e*cos(angles)); XX = RR.*cos(angles); YY = RR.*sin(angles);

% Reference circle

% Xref = a*cos(angles)-a*e; Yref = a*sin(angles);

% new orbit

RR_new = p2 ./ (1 + e2*cos(angles - deg2rad(Domega)));

XX_new = RR_new.*cos(angles);

YY_new = RR_new.*sin(angles);

rp_vec = rp2*[cosd(Domega), sind(Domega)];

ra_vec = ra2*[cosd(Domega+180), sind(Domega+180)];

% Earth

Xearth = Re*cos(angles); Yearth = Re*sin(angles);

fig = figure("Renderer","painters","Position",[10, 10, 900, 700]);

plot(XX,YY)

hold on; grid on; grid minor; box on; axis equal;

% Maneuver point

plot(r1vec_i(1), r1vec_i(2), '.r', 'MarkerSize', 20)

% Center of orbit

plot(0,0,'.k', 'MarkerSize', 15)

% Reference circle

% plot(Xref, Yref, '-m')

% Semi minor axis

% plot([-a*e, -a*e], [-b, b], '--k')

% Earth

plot(Xearth, Yearth, '-g')

plot(XX_new, YY_new, '-m')

plot([-ra, rp], [0, 0], '--k')

plot([ra_vec(1), rp_vec(1)],[ra_vec(2), rp_vec(2)], '--r')

hold off

title('PS6 P3 Orbit without Maneuver - T. Koike')

xlabel('$\hat{e}$ [km]')

ylabel('$\hat{p}$ [km]')

saveas(fig, fullfile(fdir, "p3-orbit-withManeuver.png"))

function TA = TAfromRVGamma(r, v, gamma, mu, unit)

A = r*v^2/mu;

if unit == "rad"

gamma = rad2deg(gamma);

end

num = A*cosd(gamma)*sind(gamma);

den = A*cosd(gamma)^2 - 1;

TA = atan_dbval(num/ den, unit);

end

Functions

function v = vis_viva(r, a, mu)

%{

NAME:    VIS_VIVA

AUTHOR:  TOMOKI KOIKE

INPUTS:  (1) r:  POSITION (LENGTH) ON ORBIT

(2) a:  SEMI MAJOR AXIS

(3) mu: GRAVITATIONAL PARAMETER

OUTPUTS: (1) v:  VELOCITY AT THE POSITION

DESCRIPTION: CALCULATES THE VELOCITY FOR A CERTAIN POSITION ON A

CONIC ORBIT.

%}

v = sqrt(mu * (2/r - 1/a));

end

function E = T2E_anomaly(e, theta_star, unit)

%{

NAME:    T2E_anomaly

AUTHOR:  TOMOKI KOIKE

INPUTS:  (1) e:          ECCENTRICITY

(2) theta_star: TRUE ANOMALY

(3) unit:       DEGREES OR RADIANS

OUTPUTS: (1) E:          ECCENTRIC ANOMALY

DESCRIPTION: CALCULATES THE ECCENTRIC ANOMALY FROM THE TRUE ANOMALY.

%}

ee = sqrt((1 - e) / (1 + e));

if unit == "deg"

E = 2*atand(ee * tand(theta_star / 2));

else

E = 2*atan(ee * tan(theta_star / 2));

end

end

function theta_star = E2T_anomaly(e, E, unit)

%{

NAME:    E2T_anomaly

AUTHOR:  TOMOKI KOIKE

INPUTS:  (1) e:    ECCENTRICITY

(2) E:    ECCENTRIC ANOMALY

(3) unit: DEGREES OR RADIANS

OUTPUTS: (1) theta_star: TRUE ANOMALY

DESCRIPTION: CALCULATES THE TRUE ANOMALY FROM THE ECCENTRIC ANOMALY.

%}

ee = sqrt((1 + e) / (1 - e));

if unit == "deg"

theta_star = 2*atand(ee * tand(E / 2));

if theta_star < 0

theta_star = 360 + theta_star;

end

else

theta_star = 2*atan(ee * tan(E/ 2));

if theta_star < 0

theta_star = 2*pi + theta_star;

end

end

end

function res = atan_dbval(x, unit)

if unit == "deg"

ang1 = atand(x); % -90 to 90

ang2 = 180 - (-ang1);

if ang2 > 180

ang2 = ang2 - 360;

end

else

ang1 = atan(x);

ang2 = pi - (-ang1);

if ang2 > pi

ang2 = ang2 - 2*pi;

end

end

res = [ang1, ang2];

end

function res = asin_dbval(x, unit)

if unit == "deg"

ang1 = asind(x);

if (0<=ang1 && ang1<=180)

ang2 = 180 - ang1;

elseif -90<=ang1 && ang1<0

ang2 = -ang1 - 180;

else

ang2 = 540 - ang1;

end

else

ang1 = asin(x);

if (0<=ang1 && ang1<=pi)

ang2 = pi - ang1;

elseif -pi/2<=ang1 && ang1<0

ang2 = -ang1 - pi;

else

ang2 = 3*pi - ang1;

end

end

res = [ang1, ang2];

end

function res = acos_dbval(x, unit)

if unit == "deg"

ang1 = acosd(x);

if (0<=ang1 && ang1<=180)

ang2 = -ang1;

else

ang2 = 360 - ang1;

end

else

ang1 = asin(x);

if (0<=ang1 && ang1<pi)

ang2 = -ang1;

else

ang2 = 2*pi - ang1;

end

end

res = [ang1, ang2];

end

Auxiliary MATLAB Setup File

%% Table of Constants

function planets = setup_planetary_constants()

%{

arp  : Axial Rotational Period (Rev / Day)

mer  : Mean Equatorial Radius (km)

gp   : Gravitational Parameter, mu (km^3 / s^2)

smao : Semi-Major Axis of Orbit (km)

op   : Orbital Period (s)

eo   : Eccentricity of Orbit

ioe  : Inclination of Orbit to Ecliptic (deg)

%}

% Sun

sun.arp = 0.0394011;

sun.mer = 695990;

sun.gp  = 132712440017.99;

sun.smao = NaN;

sun.op   = NaN;

sun.eo   = NaN;

sun.ioe  = NaN;

% Moon

moon.arp  = 0.0366004;

moon.mer  = 1738.2;

moon.gp   = 4902.8005821478;

moon.smao = 384400;

moon.op   = 2360592;

moon.eo   = 0.0554;

moon.ioe  = 5.16;

% Mercury

mercury.arp  = 0.0170514;

mercury.mer  = 2439.7;

mercury.gp   = 22032.080486418;

mercury.smao = 57909101;

mercury.op   = 7600537;

mercury.eo   = 0.20563661;

mercury.ioe  = 7.00497902;

% Venus

venus.arp  = 0.0041149;  % retrograde

venus.mer  = 6051.9;

venus.gp   = 324858.59882646;

venus.smao = 108207284;

venus.op   = 19413722;

venus.eo   = 0.00676399;

venus.ioe  = 3.39465605;

% Earth

earth.arp  = 1.0027378;

earth.mer  = 6378.1363;

earth.gp   = 398600.4415;

earth.smao = 149597898;

earth.op   = 31558205;

earth.eo   = 0.01673163;

earth.ioe  = 0.00001531;

% Mars

mars.arp  = 0.9747000;

mars.mer  = 3397;

mars.gp   = 42828.314258067;

mars.smao = 227944135;

mars.op   = 59356281;

mars.eo   = 0.09336511;

mars.ioe  = 1.84969142;

% Jupiter

jupiter.arp  = 2.4181573;

jupiter.mer  = 71492;

jupiter.gp   = 126712767.8578;

jupiter.smao = 778279959;

jupiter.op   = 374479305;

jupiter.eo   = 0.04853590;

jupiter.ioe  = 1.30439695;

% Saturn

saturn.arp  = 2.2522053;

saturn.mer  = 60268;

saturn.gp   = 37940626.061137;

saturn.smao = 1427387908;

saturn.op   = 930115906;

saturn.eo   = 0.05550825;

saturn.ioe  = 2.48599187;

% Uranus

uranus.arp  = 1.3921114;  % retrograde

uranus.mer  = 25559;

uranus.gp   = 5794549.0070719;

uranus.smao = 2870480873;

uranus.op   = 2652503938;

uranus.eo   = 0.04685740;

uranus.ioe  = 0.77263783;

% Neptune

neptune.arp  = 1.4897579;

neptune.mer  = 25269;

neptune.gp   = 6836534.0638793;

neptune.smao = 4498337290;

neptune.op   = 5203578080;

neptune.eo   = 0.00895439;

neptune.ioe  = 1.77004347;

% Pluto

pluto.arp  = -0.1565620;  % retrograde

pluto.mer  = 1162;

pluto.gp   = 981.600887707;

pluto.smao = 5907150229;

pluto.op   = 7830528509;

pluto.eo   = 0.24885238;

pluto.ioe  = 17.14001206;

% Return

planets.sun = sun;

planets.moon = moon;

planets.mercury = mercury;

planets.venus = venus;

planets.earth = earth;

planets.mars = mars;

planets.jupiter = jupiter;

planets.saturn = saturn;

planets.uranus = uranus;

planets.neptune = neptune;

planets.pluto = pluto;

end