College of Engineering

School of Aeronautics and Astronautics

AAE 564

System Analysis and Synthesis

Homework 1

State Space Representation of Dynamic Systems

Author:

Tomoki Koike

Supervisor:

Martin Corless

September 4th, 2020 Friday

Purdue University

West Lafayette, Indiana

Derivation:

The system equation for the double pendulum cart system is

m0+m1+m2y-m1l1cosθ1θ1-m2l2cosθ2θ2+m1l1sinθ1θ12+m2l2sinθ2θ22=u
-m1l1cosθ1y+m1l12θ1                          +               m1l1gsinθ1                 =0
-m2l2cosθ2y+m2l22θ2                         +               m2l2gsinθ2                 =0

We simulate this system of equations using MATLAB for given initial conditions and input parameters. The given initial conditions and input parameters are organized in the following table.

|  | m0 | m1 | m2 | l1 | l2 | g | u |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 2 | 1 | 1 | 1 | 1 | 1 | 0 |
| P2 | 2 | 1 | 1 | 1 | 0.99 | 1 | 0 |
| P3 | 2 | 1 | 0.5 | 1 | 1 | 1 | 0 |
| P4 | 2 | 1 | 1 | 1 | 0.5 | 1 | 0 |

|  | y | θ1 | θ2 | y | θ1 | θ2 |
| --- | --- | --- | --- | --- | --- | --- |
| IC1 | 0 | -10° | 10° | 0 | 0 | 0 |
| IC2 | 0 | 10° | 10° | 0 | 0 | 0 |
| IC3 | 0 | -90° | 90° | 0 | 0 | 0 |
| IC4 | 0 | -90.01° | 90° | 0 | 0 | 0 |
| IC5 | 0 | 100° | 100° | 0 | 0 | 0 |
| IC6 | 0 | 100.01° | 100° | 0 | 0 | 0 |
| IC7 | 0 | 179.99° | 0° | 0 | 0 | 0 |

Simulations:

P1 and IC1

P1 and IC2

P1 and IC3

P1 and IC7

P4 and IC1

P4 and IC2

P4 and IC3

P4 and IC4

Appendix

MATLAB Code

# AAE 564 HW 1

#### Author: Tomoki Koike

clear all; close all; clc;

fdir = 'C:\Users\Tomo\Desktop\studies\2020-Fall\AAE564\matlab_simulink\outputs\hw1';

set(groot, 'defaulttextinterpreter','latex');

set(groot, 'defaultAxesTickLabelInterpreter','latex');

set(groot, 'defaultLegendInterpreter','latex');

### Simulation

% Simulate this system for given initial conditions

% Initial conditions

IC1 = [0, -10, 10, 0, 0, 0];

IC2 = [0, 10, 10, 0, 0, 0];

IC3 = [0, -90, 90, 0, 0, 0];

IC4 = [0, -90.01, 90, 0, 0, 0];

IC5 = [0, 100, 100, 0, 0, 0];

IC6 = [0, 100.01, 100, 0, 0, 0];

IC7 = [0, 179.99, 0, 0, 0, 0];

% P1

m0 = 2;

m1 = 1;

m2 = 1;

l1 = 1;

l2 = 1;

g = 1;

u = 0;

t_span = linspace(0, 60, 2^10);  % time span

% opts = odeset('RelTol',1e-6,'AbsTol',1e-7);  % option for ode

% P1 and IC1, IC2, IC3, IC7

IC = [IC1; IC2; IC3; IC7];

counter = 1;

for i = [1, 2, 3, 7]

[t, q] = ode45(@(t, x) double_pendulum_system(t, x, m0, m1, m2, l1, l2, g, u), t_span, IC(counter,:));

plot_simulation(t, q, fdir, "P1&IC"+num2str(i));

counter = counter + 1;

end

% P4

m0 = 2;

m1 = 1;

m2 = 1;

l1 = 1;

l2 = 1;

g = 0.5;

u = 0;

% P4 and IC1, IC2, IC3, IC4

IC = [IC1; IC2; IC3; IC4];

counter = 1;

for i = [1, 2, 3, 4]

[t, q] = ode45(@(t, x) double_pendulum_system(t, x, m0, m1, m2, l1, l2, g, u), t_span, IC(counter,:));

plot_simulation(t, q, fdir, "P4&IC"+num2str(i));

counter = counter + 1;

end

### Functions

function dxdt = double_pendulum_system(t, x, m0, m1, m2, l1, l2, g, u)

dxdt = zeros(6, 1);  % Preallocate the derivative vector

% Set the variables

y = x(1);

theta1 = x(2);

theta2 = x(3);

y_dot = x(4);

theta1_dot = x(5);

theta2_dot = x(6);

% Create matrices to simplify the calculations

M = [      m0 + m1 + m2, -m1*l1.*cosd(theta1), -m2*l2.*cosd(theta2);

-m1*l1.*cosd(theta1),            m1*l1.^2,                   0;

-m2*l2.*cosd(theta2),                   0,            m2*l2.^2 ];

G = [(m1*l1.*sind(theta1).*theta1_dot.^2 + m2*l2.*sind(theta2).*theta2_dot.^2); m1*l1*g.*sind(theta1); m2*l2*g.*sind(theta2)];

W = [1; 0; 0];

dxdt(1) = y_dot;

dxdt(2) = theta1_dot;

dxdt(3) = theta2_dot;

dxdt(4:end) = M \ (W * u - G);

end

function plot_simulation(t, q, file_dir, file_name_str)

P_IC = split(file_name_str, "&");

% Plot position of cart vs time

fig1 = figure(1);

plot(t, q(:, 1))

title("Position of Cart Over Time "+P_IC(1)+"/"+P_IC(2)+ " - T. Koike")

xlabel('t [s]')

ylabel('y [m]')

grid on; grid minor; box on;

saveas(fig1, fullfile(file_dir, "y_"+file_name_str+".png"));

% Plot the angles of pendulums vs time

fig2 = figure(2);

plot(t, q(:, 2))

hold on

plot(t, q(:, 3))

hold off

title("Angles of Pendulums Over Time "+P_IC(1)+"/"+P_IC(2)+ " - T. Koike")

xlabel('t [s]')

ylabel('$\theta$ [deg]')

legend('$\theta_1$', '$\theta_2$')

grid on; grid minor; box on;

saveas(fig2, fullfile(file_dir, "theta1&2_"+file_name_str+".png"));

% Plot the velocity of cart vs time

fig3 = figure(3);

plot(t, q(:, 4))

title("Velocity of Cart Over Time "+P_IC(1)+"/"+P_IC(2)+ " - T. Koike")

xlabel('t [s]')

ylabel('$\dot{y}$ [m/s]')

grid on; grid minor; box on;

saveas(fig3, fullfile(file_dir, "ydot_"+file_name_str+".png"));

% Plot the angular velocities vs time

fig4 = figure(4);

plot(t, q(:, 5))

hold on

plot(t, q(:, 6))

hold off

title("Angular Velocities of Pendulums Over Time "+P_IC(1)+"/"+P_IC(2)+ " - T. Koike")

xlabel('t [s]')

ylabel('$\dot{\theta}$ [deg/s]')

legend('$\dot{\theta_1}$', '$\dot{\theta_2}$')

grid on; grid minor; box on;

saveas(fig4, fullfile(file_dir, "thetadot1&2_"+file_name_str+".png"));

end