College of Engineering

School of Aeronautics and Astronautics

AAE 564

System Analysis and Synthesis

Homework 12

State Feedback and Stabilization

Author:

Tomoki Koike

Supervisor:

Martin Corless

November 20th, 2020 Friday

Purdue University

West Lafayette, Indiana

Exercise 1

(By hand) Determine whether or not each of the following systems are controllable, stabilizable, or not stabilizable.

(a)

x1=-x1+ux2=x2+u

(b)

x1=-x1x2=x2+u

(c)

x1=x1+ux2=x2+u

(a)

The A matrix and B matrix are

A=-1001,  B=11  .

Then the controllability matrix becomes

Qc=BAB=1-111  .

The reduced echelon form of this matrix is

1-111~1001

∴rankQc=2  .

Thus, this system is controllable.

For any gain matrix K

A+BK=-1001+11k1k2=k1-1k2k1k2+1  .

Then

detλI-A+BK= 0

⇒λ2+-k1-k2 λ+k1-k2-1=0

If k1=1 and k2=-4

⇒λ2+3λ+2=0

The characteristics polynomial has all positive coefficients, so the corresponding eigenvalues have a negative real part and is asymptotically stable. Thus, this system is stabilizable.

(b)

The A matrix and B matrix are

A=-1001,  B=01  .

Then the controllability matrix becomes

Qc=BAB=0011  .

The reduced echelon form of this matrix is

0011~1100

∴rankQc=1  .

Thus, this system is uncontrollable.

The eigenvalues of this system are

detA-λI=0⇒-1-λ1-λ=0  .

The uncontrollable eigenvalue can be found by the PBH test

For λ=1

Z=rankA-λIB=rank-200001=2

For λ=-1

Z=rankA-λIB=rank000021=1≠2

Thus, the uncontrollable eigenvalue has a negative real part so this system is stabilizable.

(c)

The A matrix and B matrix are

A=1010,  B=11  .

Then the controllability matrix becomes

Qc=BAB=1111  .

The reduced echelon form of this matrix is

1111~1100

∴rankQc=1  .

Thus, this system is uncontrollable.

The eigenvalues of this system are

detA-λI=0⇒1-λ-λ=0  .

The uncontrollable eigenvalue can be found by the PBH test

For λ=0

Z=rankA-λIB=rank101101=1≠2

For λ=1

Z=rankA-λIB=rank0011-11=2

Thus, the uncontrollable eigenvalue does not have a negative real part so this system is not stabilizable.

Exercise 2

Obtain an open loop control which drives the following system from x0=-1 to x1=1.

x=x+u

From the problem we know that

x0=x0=-1

T=1:     xf=xT=1

A=1,  B=1

The controllability grammian is going to be

WcT≔0TeAtBB'eA'tdt

Wc1=01etetdt=01e2tdt=12(e2-1) =3.1945 .

Then,

x=WcT-1xf-eATx0=13.19451-e-1=1.1640 .

Then the control input will become

ut=B'eA'T-tx=e1-t1.1640

Thus, the open loop control that drives x0 to xf for a given time frame of 0,T is

ut=1.1640e1-t

Exercise 3

(By hand) Consider the system described by

x1=x1+x2+ux2=u

Obtain a state feedback controller which results in a closed loop system which is asymptotically stable about the zero state.

The A, B matrix of this system are

A=1100,  B=01

The controllability matrix is

Qc=BAB=0110

∴rankQc=2  .

This system is controllable.

Choose two asymptotically stable poles

p=-1, -2  .

Use the eigenvalue placement method to stabilize the system.

Say the feedback gains are

K=k1k2 .

Then

A+BK=1100+01k1k2=11k1k2  .

Then

detλI-A+BK= 0    ⇒    λ2+-k2-1 λ-k1+k2=0

The characteristic equation with the desired poles is

λ+1λ+2=λ2+3λ+2=0  .

Then, equate

-k2-1=3-k1+k2=2  ⇒  k1=-6,k2=-4  ⇒  K=-6-4

Exercise 4

(By hand) Consider the system described by

x1=-x2+u

x2=-x1-u

Where all quantities are scalars.

- Is this system stabilizable via state feedback?

- Does there exist a linear state feedback controller which results in closed loop eigenvalues -1, -4?

- Does there exist a linear state feedback controller which results in closed loop eigenvalues -2, -4?

In parts (b) and (c): If no controller exists, explain why; if one does exist, give an example of one.

(a)

The A matrix and B matrix are

A=0-1-10,  B=1-1  .

Then the controllability matrix becomes

Qc=BAB=11-1-1~1100⇒rankQc=1≠2.

Thus, this system is uncontrollable.

The eigenvalues of this system are

detA-λI=0⇒-1-λ1-λ=0  .

The uncontrollable eigenvalue can be found by the PBH test

For λ=1

Z=rankA-λIB=rank-1-11-1-1-1=2

For λ=-1

Z=rankA-λIB=rank1-11-11-1=1≠2

From the PBH test we know that the uncontrollable eigenvalue has a negative real part so this is stabilizable.

For a feedback gain matrix K

A+BK=0-1-10+1-1k1k2=k1k2-1-k1-1-k2  .

Then

detλI-A+BK= 0

⇒λ2+k2-k1 λ-k1+k2-1=0

(b)

If the desired poles are -1 and -4, the corresponding characteristic equation will be

λ2+5λ+4=0  .

Then from part (a) we can find the gains of the state feedback controller.

k2-k1=5-k1+k2-1=4

The augmented matrix is

-115-115~1-1-5000

So, there are infinite amount of gains that satisfy

k1-k2=-5  .

And one example is

k1=-4,  k2=1

Thus, there exists a linear state feedback controller of K=-41 .

(c)

If the desired poles are -2 and -4, the corresponding characteristic equation will be

λ2+6λ+8=0  .

Then from part (a) we can find the gains of the state feedback controller.

k2-k1=6-k1+k2-1=8

The augmented matrix is

-116-118~1-1-6002

In the second row, 2 cannot be equal to 0. Thus, there does NOT exist a linear state controller for the eigenvalues of -2, -4.

Exercise 5

(Stabilization of cart pendulum system via state feedback.) (MATLAB)

Carry out the following parameter sets P2 and P4 and equilibriums E1 and E2. Illustrate the effectiveness of your controllers with numerical simulations.

Using eigenvalue placement techniques, obtain a state feedback controller which stabilizes the nonlinear system about the equilibrium.

What is the largest value of δ (in degrees) for which your controller guarantees convergence of the closed loop system to the equilibrium for initial conditions

y,θ1,θ2,y,θ1,θ20=0,θ1e-δ,θ2e+δ,0,0,0

Where θ1e and θ2e are the equilibrium values of θ1 and θ2.

The system equation for the double pendulum cart system is

m0+m1+m2y-m1l1cosθ1θ1-m2l2cosθ2θ2+m1l1sinθ1θ12+m2l2sinθ2θ22=u
-m1l1cosθ1y+m1l12θ1                          +               m1l1gsinθ1                 =0
-m2l2cosθ2y+m2l22θ2                         +               m2l2gsinθ2                 =0

Have the system be a single output of the displacement y.

E1:      ye,θ1e,θ2e=0,0,0

E2:     ye,θ1e,θ2e=0,π,π

|  | m0 | m1 | m2 | l1 | l2 | g | u |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 2 | 1 | 1 | 1 | 1 | 1 | 0 |
| P2 | 2 | 1 | 1 | 1 | 0.99 | 1 | 0 |
| P3 | 2 | 1 | 0.5 | 1 | 1 | 1 | 0 |
| P4 | 2 | 1 | 1 | 1 | 0.5 | 1 | 0 |

| L1 | P1 | E1 |
| --- | --- | --- |
| L2 | P1 | E2 |
| L3 | P2 | E1 |
| L4 | P2 | E2 |
| L5 | P3 | E1 |
| L6 | P3 | E2 |
| L7 | P4 | E1 |
| L8 | P4 | E2 |

The Simulink model used for this is shown below,

Embedded MATLAB Block – Function (code)

function y = fcn(u, data, data1, data2)

%{

EMBEDDED MATLAB BLOCK FUNCTION

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -m1*l1*sin(u(1))*u(3)*u(3) - m2*l2*sin(u(2))*u(4)*u(4)...

- m1*g*sin(u(1))*cos(u(1)) - m2*g*sin(u(2))*cos(u(2))...

+ u(5);

den = m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2;

y = num / den;

end

Embedded MATLAB Block – Function1 (code)

function y = fcn(u, data, data1, data2)

%{

EMBEDDED MATLAB BLOCK FUNCTION1

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -(m1*l1*cos(u(1))*sin(u(1))*u(3)*u(3) + m2*l2*cos(u(1))*sin(u(2))*u(4)*u(4))...

+ m2*g*(sin(u(1))*cos(u(2))^2 - cos(u(1))*sin(u(2))*cos(u(2)))...

- (m0 + m1 + m2)*g*sin(u(1)) + u(5)*cos(u(1));

den = l1*(m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2);

y = num / den;

end

Embedded MATLAB Block – Function2 (code)

function y = fcn(u, data, data2, data1)

%{

EMBEDDED MATLAB BLOCK FUNCTION2

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -(m1*l1*cos(u(2,1))*sin(u(1))*u(3)*u(3) + m2*l2*cos(u(2))*sin(u(2))*u(4)*u(4))...

+ m1*g*(sin(u(2))*cos(u(1))^2 - cos(u(2))*sin(u(1))*cos(u(1)))...

- (m0 + m1 + m2)*g*sin(u(2)) + u(5)*cos(u(2));

den = l2*(m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2);

y = num / den;

end

For the conditions E1 and E2, we set the initial conditions of the integrator block of y, θ1, and θ2 correspondingly to ye, θ1e, θ2e; like in the following windows,

The feedback control system is the following Simulink model

“hw12_feedbackControl”

Procedure:

- Linearize the system for P# and E# and obtain the A, B, C, D matrices for corresponding equilibrium conditions

- Define the system requirements – eigenvalues to be placed

- Compute the feedback gain K with eigenvalue placement

- Plot the response of the controlled system with the system above

L3 (P2 & E1):

1.

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 -0.5000 -0.5000 0 0 0 0 -1.5000 -0.5000 0 0 0 0 -0.5051 -1.5152 0 0 0 | B = 6×1 0 0 0 0.5000 0.5000 0.5051 |
| --- | --- |
| C = 1×6 1 0 0 0 0 0 | D = 0 |

2.

p = -23/15 + 1207/363i -11/50 + 0i -3 + 0i -23/15 - 1207/363i -3/2 + 0i -9/2 + 0i

3.

K = 1×6 104 × 0.0118 6.2267 -6.1632 0.0709 -2.7555 2.6601

4.

δ=180°

L4 (P2 & E2):

1.

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 -0.5000 -0.5000 0 0 0 0 1.5000 0.5000 0 0 0 0 0.5051 1.5152 0 0 0 | B = 6×1 0 0 0 0.5000 -0.5000 -0.5051 |
| --- | --- |
| C = 1×6 1 0 0 0 0 0 | D = 0 |

2.

p = -23/15 + 1207/363i -11/50 + 0i -3 + 0i -23/15 - 1207/363i -3/2 + 0i -9/2 + 0i

3.

K = 1×6 105 × 0.0012 1.1301 -1.1191 0.0071 1.1914 -1.1727

4.

δ=0.15°

L7 (P4 & E1):

1.

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 -0.5000 -0.5000 0 0 0 0 -1.5000 -0.5000 0 0 0 0 -1.0000 -3.0000 0 0 0 | B = 6×1 0 0 0 0.5000 0.5000 1.0000 |
| --- | --- |
| C = 1×6 1 0 0 0 0 0 | D = 0 |

2.

p = -23/15 + 1207/363i -11/50 + 0i -3 + 0i -23/15 - 1207/363i -3/2 + 0i -9/2 + 0i

3.

K = 1×6 59.7291 627.9728 -279.9391 358.1599 -278.3304 -27.6281

4.

δ=180°

L8 (P4 & E2):

1.

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 -0.5000 -0.5000 0 0 0 0 1.5000 0.5000 0 0 0 0 1.0000 3.0000 0 0 0 | B = 6×1 0 0 0 0.5000 -0.5000 -1.0000 |
| --- | --- |
| C = 1×6 1 0 0 0 0 0 | D = 0 |

2.

p = -23/15 + 1207/363i -11/50 + 0i -3 + 0i -23/15 - 1207/363i -3/2 + 0i -9/2 + 0i

3.

K = 1×6 103 × 0.0597 1.1405 -0.6133 0.3582 1.2035 -0.4349

4.

δ=0.15°

Exercise 6

(By hand) Consider the discrete time system

xk+1=x1k+x2k

x2k+1=x2k+uk

Obtain a state feedback controller which always drives the state of this system to zero in at most two steps.

The state matrices become

Ad=1101,  Bd=01

The controllability matrix is

Qc=BAB=0111~(1001)

∴rankQc=2  .

This system is controllable.

Define a gain K so that u=-Kx.

Ad+BdK=0111+01k1k2=11k1k2+1

Take the determinant, and it becomes

λ2+-k2-2 λ-k1+k2+1=0

If the system goes to zero in two steps (or two seconds with a sampling time of one second), the Cayley-Hamilton Theorem implies that the following should be satisfied

Ad2=0

Thus, the characteristic polynomial of the desired poles become

s2=0

Thus, we solve

-k2-2=0

-k1+k2+1=0

Which gives us

K=-1-2  .

Exercise 7

For the system described by

2q1+q2-q2=u1

q1+2q2-q1=u2

Obtain a feedback controller generating u1 and u2 which stabilizes this system. Assume that q1,q2,q1,q2 can be measured. You can use MATLAB for some of this.

Manipulating the given equations, we get the following relations

q1=-13q1+23q2+23u1-13u2

q2=-13q1+23q2-13u1+23u2

Let, x1≔q1,x2≔q2,x3≔q1,x4≔q2. Then A, B matrices of this system are

A=00100001-132300-132300,  B=000023-13-1323

The controllability matrix becomes

Qc=BABA2BA3B=0023-1300-495900-132300-495923-1300-495900-132300-495900

The row reduced echelon form of this matrix is

~1000-4353000100-435300001000-4353000100-4353

Thus,

rankQc=4

And this system is controllable, so we can choose any poles to control this system. The arbitrary poles that we choose are

λ=-1.25+2.5j-1.25-2.5j-0.3536-0.7071 .

Now we find the controller gains using the Brogan’s Algorithm

Step 1:

Find

ϕ=xIn×n-A-1

⇒-3 x2-2x-3 x3-2x-3 x3-3 x2-2x2-3 x4-2x2-3 x41x-3 x3-3 x2+1x-3 x31x2-3 x4-3 x2+1x2-3 x4-13 x2-123 x2-1-3 x2-2x-3 x3-2x-3 x3-13 x2-123 x2-11x-3 x3-3 x2+1x-3 x3

Step 2:

Compute

ψ=ϕB

⇒0.6667x2-3 x4-0.6667 3 x2-2x2-3 x40.3333 3 x2-2x2-3 x4-1.3333x2-3 x40.3333 3 x2+1x2-3 x4+0.6667x2-3 x4-0.6667 3 x2+1x2-3 x4-0.3333x2-3 x40.6667x-3 x3-0.6667 3 x2-2x-3 x30.3333 3 x2-2x-3 x3-1.3333x-3 x30.3333 3 x2+1x-3 x3+0.6667x-3 x3-0.6667 3 x2+1x-3 x3-0.3333x-3 x3

Step 3:

Calculate

ψ=ψ1λ1ψ1λ2ψ2λ3ψ2λ4

Where ψ1x,ψ2x correspond to the columns of ψ.

ψ=-0.0494+0.0751 j-0.0494-0.0751 j0.50003.8362e-060.0274-0.0273 j0.0274+0.0273 j1.50000.5000-0.1261-0.2175 j-0.1261+0.2175 j-0.5000-5.4251e-060.0339+0.1025 j0.0339-0.1025 j-1.5000-0.7071

Step 4:

Find the gains with

K=-Eψ-1

Where

E=11000011

Thus,

K=-11000011-0.0494+0.0751 j-0.0494-0.0751 j0.50003.8362e-060.0274-0.0273 j0.0274+0.0273 j1.50000.5000-0.1261-0.2175 j-0.1261+0.2175 j-0.5000-5.4251e-060.0339+0.1025 j0.0339-0.1025 j-1.5000-0.7071

K=8.5075-8.30061.2140-5.86940.62253.72831.65594.0505

And

Acl=A-BK=00100001-5.79757.4432-0.25745.26312.0875-4.5858-0.6993-4.6568.

The eigenvalues become

eigAcl=-1.4142-1-1.2500-2.5000 j-1.2500+2.5000 j

Exercise 8

Consider the system described by

x1=x1+u

x2=-x2+2u

Obtain (by hand) a state-feedback controller (it will not be a static controller) which always results in the state of the closed loop going to zero in at most 2 secs. Illustrate your results with a simulation.

The A and B matrices of this continuous time system is

Ac=100-1,  Bc=12

The discrete time state space becomes

Ad=eAcT,  Bd=0TeAc τdτBc

Let the sampling time, T be 1 second. Then

Ad=eAc,  Bd=AdBc

⇒Ad=2.7183000.3679,  Bd=1.71831.2642

The controllability matrix is

Qc=BdAdBd=1.71834.67081.26420.4651~1001

∴rankQc=2  .

This system is controllable.

Define a gain K so that u=-Kx.

Ad+BdK=2.7183000.3679+1.71831.2642k1k2=1.7183 k1+2.71831.7183 k21.2642 k11.2642 k2+0.3679

Take the determinant, and it becomes

λ2+-1.7183 k1-1.2642 k2-3.0862 λ+0.6321 k1+3.4366 k2+1.0000=0

If the system goes to zero in two steps (or two seconds with a sampling time of one second), the Cayley-Hamilton Theorem implies that the following should be satisfied

Ad2=0

Thus, the characteristic polynomial of the desired poles become

s2=0

Thus, we solve

-1.7183 k1-1.2642 k2-3.0862=0

0.6321 k1+3.4366 k2+1.0000=0

Solving these linear equations, we obtain

k1=-1.8296,  k2=0.0455

K=[-1.82960.0455]

Now we create the following Simulink to simulate the system.

Where the state space block has

And

x0=10

The input is

The discrete output is

The continuous output is