College of Engineering

School of Aeronautics and Astronautics

AAE 564

System Analysis and Synthesis

Homework 13

Output Feedback and Lyapunov Theory

Author:

Tomoki Koike

Supervisor:

Martin Corless

December 4th, 2020 Friday

Purdue University

West Lafayette, Indiana

Exercise 1

Determine whether or not each of the following systems are observable, detectable, or not detectable.

(a)

x1=-x1x2=x2+uy=x1+x2

(b)

x1=-x1x2=x2+uy=x1

(c)

x1=x1x2=x2+uy=x1+x2

(a)

The state matrices are

A=-1001,  B=01

C=11,  D=0  .

The observability matrix

Qo=CCA=11-11~1001⇒rankQo=2  .

This system is observable, and therefore, this system is detectable.

(b)

The state matrices are

A=-1001,  B=01

C=10,  D=0  .

The observability matrix

Qo=CCA=10-10~1000⇒rankQo=1 < 2  .

This system is unobservable. Next, we have to find the unobservable eigenvalues with the PBH test. The eigenvalues are

detA-λI=λ2-1=0⇒λ=±1  .

If λ= -1,

Z=A-λ IC=000210=100100⇒rankZ=2  .

This eigenvalue is observable.

If λ=1,

Z=A-λ IC=-200010=100000⇒rankZ=1 < 2  .

This eigenvalue is unobservable.

The unobservable eigenvalue has a positive real part, and therefore, this system is not detectable.

(c)

The state matrices are

A=1001,  B=01

C=11,  D=0  .

The observability matrix

Qo=CCA=10-10~1000⇒rankQo=1 < 2  .

This system is unobservable. Next, we have to find the unobservable eigenvalues with the PBH test. The eigenvalues are

detA-λI=λ2-2λ+1=0⇒λ=1  .

If λ=1,

Z=A-λ IC=000011=110000⇒rankZ=1 < 2  .

This eigenvalue is unobservable.

The unobservable eigenvalue has a positive real part, and therefore, this system is not detectable.

Exercise 2

Consider the system described by

x1=-x2+ux2=-x1-uy=x1-x2

Where all quantities are scalars.

- Is this system observable?

- Is this system detectable?

- Does there exist an asymptotic state estimator for this system? If an estimator does not exist, explain why; if one does exist, give an example of one.

- If the answer to part (c) is yes, illustrate the effectiveness of your observer with a simulation.

(a)

The state matrices are

A=0-1-10,  B=1-1

C=1-1,  D=0  .

The observability matrix

Qo=CCA=1-11-1~1-100⇒rankQo=1 < 2  .

This system is unobservable.

(b)

The eigenvalues are

detA-λI=λ2-1=0⇒λ=±1  .

If λ= -1,

Z=A-λ IC=1-1-111-1=1-10000⇒rankZ=1  .

This eigenvalue is unobservable.

If λ=1,

Z=A-λ IC=-1-1-1-11-1=100100⇒rankZ=2   .

This eigenvalue is unobservable.

The unobservable eigenvalue has a negative real part, and therefore, this system is detectable.

(c)

Let

L=l1l2  .

we have

A+LC=l1-l1-1l2-1-l2  .

Hence,

detsI-A-LC=s-l1l1+1-l2+1s+l2=s-l1s+l2-l1+1-l2+1

⇒s2+l2-l1s-l1+l2+1=0  .

And A+LC is asymptotically stable if

l2-l1>0       and      -l1+l2+1>0     ⇒      l2>l1  .

Thus, there exists an asymptotic state estimator.

An example of an asymptotic observer is then given by

x1=2l1x1-2l1+1x2+u-l1yx2=2l2-1x1-2l2x2-u-l2y

If l1=1,l2=2

x1=2x1-3x2+u-yx2=3x1-4x2-u-2y

This is an example of an asymptotic observer.

(d)

Using the following Simulink model

We define

L=12  .

And simulate the model. Then x=x-x becomes

And the output, y being

Since the difference between the estimated states and actual states are zero we can see that the observer is effective.

Exercise 3

Consider the system,

x1=x2+u1x2=u2y=x1

where all quantities are scalar. Obtain (by hand) an output feedback controller which results in an asymptotically stable closed loop system.

The state matrices are

A=0100,  B=1001

C=10,  D=0  .

The observability matrix

Qo=CCA=1001~1001⇒rankQo=2  .

This system is observable, and thus detectable.

The controllability matrix

Qc=AAB=10010100⇒rankQc=2.

This system is controllable, and thus stabilizable.

Let the state feedback and observer matrix be

K=k1k2k3k4

L=l1l2

The closed loop system can be characterized as

x=A+BK+LC+DKx-Lyu=Kx

Now,

Ac=A+BK+LC+DK=k1+l1k2+1k3+l2k4

Bc=-L=-l1l2

Cc=K=k1k2k3k4 .

Set our desired poles as p = -2, -5.

We first find K for these desired poles using the Brogan’s algorithm

Find

ϕ=xIn×n-A-1=1x1x201x  .

Compute

ψ=ϕB=1x1x201x  .

Calculate

ψ=ψ1λ1ψ2λ2=-0.50.040-0.2

Where ψ1x,ψ2x correspond to the columns of ψ.

Find the gains with

K=Eψ-1

Where

E=1001  .

K=-2-0.040-5  .

Then,

Acl=A+BK=-20.60-5  ⇒  eigAcl=-2,-5  .

Next, for the same desired poles we will find the observer gains L.

The characteristic equation for the desired poles is

CE≔s+2s+5=s2+7s+10  .

Now,

detsI-A'+C'L'=s2-l1s-l2  .

Thus,

-l1=7-l2=10  ⇒   l1=-7l2=-10

Hence,

K=-2-0.040-5,  L=-7-10

And

Ac=A+BK+LC+DK=-90.6-10-5

Bc=-L=710

Cc=K=-2-0.040-5 .

Now we check that it is asymptotically stable

eigAc=-7±1.4142j  .

It is asymptotically stable.

Thus, the output feedback controller is

x=-90.6-10-5x+710yu=-2-0.040-5x

Exercise 4

Consider the system

x1=-x1+x3x2=ux3=x2y=x3

with scalar control input u and scalar measured output y.

- Obtain (by hand) an observer-based output feedback controller which results in an asymptotically stable closed loop system.

- Can all the eigenvalues of the closed loop system be arbitrarily placed?

(a)

The state matrices are

A=-101000010,  B=010

C=001,  D=0  .

The observability matrix

Qo=CCACA2=001010000~010001000⇒rankQo=2 < 3  .

This system is unobservable.

detsI-A=s3+s2  ⇒  eigA=0,-1

If λ=-1,

Zo=λ I-AC=001010011001~010001000000⇒rankZo=2<3

This eigenvalue is unobservable.

If λ=0,

Zo=λ I-AC=-101000010001~100010001000⇒rankZo=3

This eigenvalue is observable. Now, since the unobservable eigenvalue has a negative real part, this system is detectable.

The controllability matrix

Qc=AABA2B=001100010⇒rankQc=3.

This system is controllable, and thus stabilizable.

Let the state feedback and observer matrix be

K=k1k2k3

L=l1l2l3  .

Now,

detsI-A-BK=s3+1-k2 s2+-k2-k3 s-k1-k3

The eigenvalues become asymptotically stable if all the coefficients are positive.

1-k2>0-k2-k3>0-k1-k3>0  .

detsI-A-LC=s3+1-l3 s2+-l2-l3 s-l2

The eigenvalues become asymptotically stable if all the coefficients are positive.

1-l3>0-l2-l3>0-l2>0  .

For the L matrix we find the positive combinations of a,b,c that satisfy the following

1-l3=a-l2-l3=b-l2=c

Which means that a,c can be arbitrary but b must satisfy b=a+c-1. Thus, we choose

a=5,  b= 10,  c=6

Where

l2=-6,  l3=-4

And l1 can be any value so we select

l1=-1  .

Corresponding to the coefficients a,b,c, the characteristic equation becomes

CE≔s3+5s2+10s+6

Then,

1-k2=5-k2-k3=10-k1-k3=6 ⇒ k1=0k2=-4k3=-6  .

Hence,

K=0-4-6,  L=-1-6-4

The closed loop system can be characterized as

x=A+BK+LC+DKx-Lyu=Kx

And

Ac=A+BK+LC+DK=-1000-4-1201-4

Bc=-L=164

Cc=K=0-4-6.

Now we check that it is asymptotically stable

eigAc=-4±3.464j, -1  .

It is asymptotically stable.

Thus, the output feedback controller is

x=-1000-4-1201-4x+164yu=0-4-6x  .

(b)

From the relation in part (a),

detsI-A-LC=s3+1-l3 s2+-l2-l3 s-l2

1-l3>0-l2-l3>0-l2>0  .

We can see that the matrix representing these three inequality equations can be represented by the matrix

00-110-1-100-100~010000100001  .

This shows that the eigenvalues have to be selected so that the following relationship is satisfied

1-l3=a-l2-l3=b-l2=c

Thus, the desired poles CANNOT be selected arbitrarily.

Exercise 5

Consider the system,

x1=x2x2=x1+uy=x1

where all quantities are scalar. Obtain (by hand) an output feedback controller which results in an asymptotically stable closed loop system.

The state matrices are

A=0110,  B=01

C=10,  D=0  .

The observability matrix

Qo=CCA=1001⇒rankQo=2   .

This system is observable, and therefore, detectable.

The controllability matrix

Qc=AAB=0110⇒rankQc=2.

This system is controllable, and thus stabilizable.

Let the state feedback and observer matrix be

K=k1k2

L=l1l2  .

Let the desired poles be p=-2, -3, the corresponding characteristic equation becomes

CE≔s+2s+3=s2+5s+6

Now,

detsI-A-BK=s2-k2 s-k1-1

detsI-A-LC=s2-l1 s-l2-1

Hence,

K=-7-5,  L=-5-7  .

The closed loop system can be characterized as

x=A+BK+LC+DKx-Lyu=Kx

And

Ac=A+BK+LC+DK=-51-13-5

Bc=-L=57

Cc=K=-7-5.

Now we check that it is asymptotically stable

eigAc=-5±3.6056j  .

It is asymptotically stable.

Thus, the output feedback controller is

x=-51-13-5x+57yu=-7-5x  .

Exercise 6

(Stabilization of cart pendulum system via output feedback.) consider the cart pendulum system with the displacement y as the measured output. Carry out the following for parameter P4 and equilibriums E1 and E2. Illustrate the effectiveness of your controllers with numerical simulations.
using eigenvalue placement techniques, obtain a output feedback controller which stabilizes the nonlinear system about the equilibrium. 
What is the largest value of δ (in degrees) for which your controller guarantees convergence of the closed loop system to the equilibrium for initial condition

y,θ1,θ2,y,θ1,θ20=0,θ1e-δ,θ2e+δ,0,0,0

Where θ1e and θ2e are the equilibrium values of θ1 and θ2.

The system equation for the double pendulum cart system is

m0+m1+m2y-m1l1cosθ1θ1-m2l2cosθ2θ2+m1l1sinθ1θ12+m2l2sinθ2θ22=u
-m1l1cosθ1y+m1l12θ1                          +               m1l1gsinθ1                 =0
-m2l2cosθ2y+m2l22θ2                         +               m2l2gsinθ2                 =0

Have the system be a single output of the displacement y.

E1:      ye,θ1e,θ2e=0,0,0

E2:     ye,θ1e,θ2e=0,π,π

|  | m0 | m1 | m2 | l1 | l2 | g | u |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 2 | 1 | 1 | 1 | 1 | 1 | 0 |
| P2 | 2 | 1 | 1 | 1 | 0.99 | 1 | 0 |
| P3 | 2 | 1 | 0.5 | 1 | 1 | 1 | 0 |
| P4 | 2 | 1 | 1 | 1 | 0.5 | 1 | 0 |

| L1 | P1 | E1 |
| --- | --- | --- |
| L2 | P1 | E2 |
| L3 | P2 | E1 |
| L4 | P2 | E2 |
| L5 | P3 | E1 |
| L6 | P3 | E2 |
| L7 | P4 | E1 |
| L8 | P4 | E2 |

The nonlinear Simulink model used for this is shown below,

Embedded MATLAB Block – Function (code)

function y = fcn(u, data, data1, data2)

%{

EMBEDDED MATLAB BLOCK FUNCTION

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -m1*l1*sin(u(1))*u(3)*u(3) - m2*l2*sin(u(2))*u(4)*u(4)...

- m1*g*sin(u(1))*cos(u(1)) - m2*g*sin(u(2))*cos(u(2))...

+ u(5);

den = m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2;

y = num / den;

end

Embedded MATLAB Block – Function1 (code)

function y = fcn(u, data, data1, data2)

%{

EMBEDDED MATLAB BLOCK FUNCTION1

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -(m1*l1*cos(u(1))*sin(u(1))*u(3)*u(3) + m2*l2*cos(u(1))*sin(u(2))*u(4)*u(4))...

+ m2*g*(sin(u(1))*cos(u(2))^2 - cos(u(1))*sin(u(2))*cos(u(2)))...

- (m0 + m1 + m2)*g*sin(u(1)) + u(5)*cos(u(1));

den = l1*(m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2);

y = num / den;

end

Embedded MATLAB Block – Function2 (code)

function y = fcn(u, data, data2, data1)

%{

EMBEDDED MATLAB BLOCK FUNCTION2

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -(m1*l1*cos(u(2,1))*sin(u(1))*u(3)*u(3) + m2*l2*cos(u(2))*sin(u(2))*u(4)*u(4))...

+ m1*g*(sin(u(2))*cos(u(1))^2 - cos(u(2))*sin(u(1))*cos(u(1)))...

- (m0 + m1 + m2)*g*sin(u(2)) + u(5)*cos(u(2));

den = l2*(m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2);

y = num / den;

end

For the conditions E1 and E2, we set the initial conditions of the integrator block of y, θ1, and θ2 correspondingly to ye, θ1e, θ2e; like in the following windows,

The system state space matrices and poles computed from the linearization for L7 and L8 are the following

L7 (P4 & E1):

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 -0.5000 -0.5000 0 0 0 0 -1.5000 -0.5000 0 0 0 0 -1.0000 -3.0000 0 0 0 | B = 6×1 0 0 0 0.5000 0.5000 1.0000 |
| --- | --- |
| C = 1×6 1 0 0 0 0 0 | D = 0 |

eigVal = 6×1 complex 0.0000 + 0.0000i 0.0000 + 0.0000i 0.0000 + 1.8113i 0.0000 - 1.8113i -0.0000 + 1.1042i -0.0000 - 1.1042i

L8 (P4 & E2):

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 -0.5000 -0.5000 0 0 0 0 1.5000 0.5000 0 0 0 0 1.0000 3.0000 0 0 0 | B = 6×1 0 0 0 0.5000 -0.5000 -1.0000 |
| --- | --- |
| C = 1×6 1 0 0 0 0 0 | D = 0 |

eigVal = 6×1 0 0 -1.8113 -1.1042 1.8113 1.1042

Using the following MATLAB code we can simulate the nonlinear system controlled by the output feedback controller. The K and L gains are computed using the pole placement with arbitrarily selected poles.

close all; clear all; clc;

set(groot, 'defaulttextinterpreter','latex');

set(groot, 'defaultAxesTickLabelInterpreter','latex');

set(groot, 'defaultLegendInterpreter','latex');

addpath(genpath("C:\Users\Tomo\Desktop\studies\2020-Fall\AAE564\matlab_simulink"))

% System requirements

p = [-1, -1.22, -1.5, -2, -2.3, -2.7];

global m l g ye theta1e theta2e

warning('off');

param_combo = ["L7","L8"]; %["L7","L8"]

for i = 1:numel(param_combo)

define_params(param_combo(i));

xe = trim('db_pend_cart_lin');

[A, B, C, D] = linmod('db_pend_cart_lin',xe);

lin_sys(i).Amat = A;

lin_sys(i).Bmat = B;

lin_sys(i).Cmat = C;

lin_sys(i).Dmat = D;

% Compute the gains

K    = -place(A, B, p);

lin_sys(i).K = K;

L    = -place(A', C', p)';

lin_sys(i).L = L;

% Output feedback matrices

Ac = A + B*K + L*C;

Bc = -L;

[B_rows, B_cols] = size(Bc);

Cc = K;

[C_rows, C_cols] = size(Cc);

Dc = zeros(C_rows, B_cols);

ICc = zeros(B_rows, 1);

% Plotting

% Initialize figure

fig = figure('Renderer',"painters", 'Position', [10 10 900 1000]);

delta_max = "0";

inc_history = [];

while true

% delta = linspace(0,deg2rad(str2double(delta_max(i))),50)

delta = str2double(delta_max);

% Initial conditions

dyi = 0;

u = 0;

yi = ye + dyi;

theta1i = theta1e - deg2rad(delta);

theta2i = theta2e + deg2rad(delta);

IC_ss = [yi, theta1i, theta2i, 0, 0, 0];

% Simulate

simout = sim('db_pend_cart_lin_outputFeedback');

lin_sys(i).simout = simout;

% Plot

time = simout.tout;

data = simout.res.signals.values;

y = data(:,1);

theta1 = data(:,2);

theta2 = data(:,3);

if i == 1

if abs(theta1(end)-theta1e) > 0.1 || abs(theta2(end)-theta2e) > 0.1

break;

elseif abs(theta1(end)-theta1e) > 0.08 || abs(theta2(end)-theta2e) > 0.08

inc = 0.01;

elseif abs(theta1(end)-theta1e) > 0.05 || abs(theta2(end)-theta2e) > 0.05

inc = 0.1;

elseif abs(theta1(end)-theta1e) > 0.01 || abs(theta2(end)-theta2e) > 0.01

inc = 0.5;

elseif abs(theta1(end)-theta1e) > 0.001 || abs(theta2(end)-theta2e) > 0.001

inc = 0.8;

else

inc = 1;

end

else

if abs(theta1(end)-theta1e) > 0.1 || abs(theta2(end)-theta2e) > 0.1

break;

else

inc = 0.01;

end

end

subplot(3,1,1)

grid on; grid minor; box on;

plot(time,y)

hold on; grid on; grid minor; box on;

ylabel('y [m]')

subplot(3,1,2)

grid on; grid minor; box on;

plot(time,theta1)

hold on; grid on; grid minor; box on;

ylabel('$\theta_1$ [rad]')

subplot(3,1,3)

grid on; grid minor; box on;

plot(time,theta2)

hold on; grid on; grid minor; box on;

ylabel('$\theta_2$ [rad]')

delta_max = num2str(str2double(delta_max) + inc);

inc_history = [inc_history, inc];

end

hold off;

xlabel('time, [sec]')

line1 = param_combo(i)+' Time Histories for Output Feedback Controlled';

delta_char = compose("%d", str2double(delta_max));

line2 = 'Cart Pendulum System for $\delta\in[0,'+delta_char+'^{\circ}]$ - T. Koike';

title_string = {line1,line2};

sgtitle(title_string)

saveas(fig, 'p6_'+param_combo(i)+'.png');

end

function define_params(L)

% Function to define parameters

global m l g ye theta1e theta2e

if L == "L1"

m = [2,1,1]; l = [1,1]; g = 1; % P1

ye = 0; theta1e = 0; theta2e = 0; % E1

elseif L == "L2"

m = [2,1,1]; l = [1,1]; g = 1; % P1

ye = 0; theta1e = pi; theta2e = pi; % E2

elseif L == "L3"

m = [2,1,1]; l = [1,0.99]; g = 1; % P2

ye = 0; theta1e = 0; theta2e = 0; % E1

elseif L == "L4"

m = [2,1,1]; l = [1,0.99]; g = 1; % P2

ye = 0; theta1e = pi; theta2e = pi; % E2

elseif L == "L5"

m = [2,1,0.5]; l = [1,1]; g = 1; % P3

ye = 0; theta1e = 0; theta2e = 0; % E1

elseif L == "L6"

m = [2,1,0.5]; l = [1,1]; g = 1; % P3

ye = 0; theta1e = pi; theta2e = pi; % E2

elseif L == "L7"

m = [2,1,1]; l = [1,0.5]; g = 1; % P4

ye = 0; theta1e = 0; theta2e = 0; % E1

elseif L == "L8"

m = [2,1,1]; l = [1,0.5]; g = 1; % P4

ye = 0; theta1e = pi; theta2e = pi; % E2

else

print('error: did not match any')

end

end

We implement an output feedback controller by finding the K and L gains using eigenvalue placement. The controller is defined as

x=A+BK+LC+DKx-Lyu=Kx

and the poles are

p=-1-1.22-1.5-2-2.3-2.7  .

The Simulink model used for the output feedback controlled system is

The simulation results are the following

L7 (P4 & E1):

The results show that maxδ=48° .

L8 (P4 & E2):

The results show that maxδ=1.98° .

Exercise 7

Using the Lyapunov equation determine (by hand) whether or not the system x=Ax is asymptotically stable for each one of the following A matrices.

(a)

-120-1

(b)

-1 201

(c)

1201

Check your answers using the MATLAB command lyap.

(a)

Let a Hermitian matrix P be

P=p11p12p12p22  .

Say

Q = 1-1-12  .

Since,

eigQ=0.3820,   2.6180⇒Q>0  .

Then from the Lyapunov equation

PA+A'P+Q=0

⇒p11p12p12p22-120-1+-102-1p11p12p12p22+1-1-12

⇒1-2 p11=02 p11-2 p12-1=02 p11-2 p12-1=04 p12-2 p22+2=0

Solving these equations, we get

P=p11p12p12p22=0.5001  .

eigP=0.5,   1⇒P>0  .

Thus, the system x=Ax is asymptotically stable.

MATLAB Verification:

% verify

A = [-1, 2; 0, -1];

Q = [1, -1; -1, 2];

P = lyap(A', Q)

eigVal = eig(P)

| P = 2×2 0.5000 0 0 1.0000 | eigVal = 2×1 0.5000 1.0000 |
| --- | --- |

(b)

Let a Hermitian matrix P be

P=p11p12p12p22  .

Say

Q = 1-1-12  .

Since,

eigQ=0.3820,   2.6180⇒Q>0  .

Then from the Lyapunov equation

PA+A'P+Q=0

⇒p11p12p12p22-1 201+-1 021p11p12p12p22+1-1-12

⇒1-2 p11=02 p11-1=02 p11-1=04 p12+2 p22+2=0

Solving these equations, we get

P=p11p12p12p22=0.5-0.5-0.50  .

eigP=-0.3090,   0.8090⇒P is not positive definite  .

Thus, the system x=Ax is NOT asymptotically stable.

MATLAB Verification:

% verify

A = [-1, 2; 0, 1];

Q = [1, -1; -1, 2];

P = lyap(A', Q)

Error using lyap (line 73)
The solution of this Lyapunov equation does not exist or is not unique.

(c)

Let a Hermitian matrix P be

P=p11p12p12p22  .

Say

Q = 1-1-12  .

Since,

eigQ=0.3820,   2.6180⇒Q>0  .

Then from the Lyapunov equation

PA+A'P+Q=0

⇒p11p12p12p221201+1021p11p12p12p22+1-1-12

⇒2 p11+1=02 p11+2 p12-1=02 p11+2 p12-1=04 p12+2 p22+2=0

Solving these equations, we get

P=p11p12p12p22=-0.511-3  .

eigP=-0.3090,   -0.1492⇒P is not positive definite  .

Thus, the system x=Ax is NOT asymptotically stable.

MATLAB Verification:

% verify

A = [1, 2; 0, 1];

Q = [1, -1; -1, 2];

P = lyap(A', Q)

eigVal = eig(P)

| P = 2×2 -0.5000 1.0000 1.0000 -3.0000 | eigVal = 2×1 -3.3508 -0.1492 |
| --- | --- |

Exercise 8

Consider the system with disturbance input w and performance output z described by

x1=-x1+x2+wx2=-x1-4x2+2wz=x1  .

Using an appropriate Lyapunov equation, determine (by hand)

0∞zt2dt

for each of the following situations.

(a)

w=0     and    x0=1, 0  .

(b)

wt=δt      and     x0=0  .

From the given system we know that

A=-11-1-4,  B=12,  C=10,  D=0

Then

Q=C'C=1010=1000  .

Let

P=p11p12p12p22  .

Then solving the Lyapunov equation we get

PA+A'P+Q=0

⇒1-2 p12-2 p11=0p11-5 p12-p22=0p11-5 p12-p22=02 p12-8 p22=0

⇒p11=0.4200,  p12=0.0800,  p22=0.0200

P=0.42000.08000.08000.0200  .

eigP=0.0046,   0.4354⇒P is positive definite  .

(a)

With w=0 and x0=1, 0'

0∞zt2dt=x0'Px0=1, 00.42000.08000.08000.020010=0.4200  .

(b)

With x0=0 and wt=δt

0∞zt2dt=B'PB=120.42000.08000.08000.020012=0.8200  .