College of Engineering

School of Aeronautics and Astronautics

AAE 564

System Analysis and Synthesis

Homework 7

Input/Output Responses of LTI Systems

Author:

Tomoki Koike

Supervisor:

Martin Corless

October 16th, 2020 Friday

Purdue University

West Lafayette, Indiana

Exercise 1

Consider the differential equation

x=Ax

where A is a square matrix. Show that if A has j2π as an eigenvalue, then there is a nonzero initial state x0 such that the equation has a solution x which satisfies x1=x0=x0.

If the given equation has an eigenvalue of

λ= j2π

the complex conjugate is also an eigenvalue of the matrix A. Thus, we know that

λ= ±j2π

From this we can deduce the solution of this equation to be

eλtv=cos2πt+jsin2πtσ+jω    ∵v = σ+jω

For

x1=cos2π+jsin2πσ+jω=σ+jω=x0

x0=cos0+jsin0σ+jω=σ+jω=x0

∴x1=x0=x0

q.e.d

Exercise 2

Compute eAt at t=ln2 for

A=0110

using all four methods mentioned in the notes.

Method 1: MATLAB “expm()”

% Problem 2

% Method 1 expm()

A = [0, 1; 1, 0];

t = log(2);

eA1 = expm(A*t);

The result is

eA1 = 2×2 1.2500 0.7500 0.7500 1.2500

Method 2: Numerical Simulation

Using ode45 we accomplish this. Following is the function

function dx = numSim(t, x, A)

dx = A*x;

end

and the following is the MATLAB script

tspan = 0:0.1:10;

A = [0, 1; 1, 0];

x0 = [1, 0];

res = ode45(@(t,x) numSim(t,x,A), tspan, x0)

E1 = deval(res, log(2))

x0 = [0, 1];

res = ode45(@(t,x) numSim(t,x,A), tspan, x0)

E2 = deval(res, log(2))

This gives us the following result

| E1 = 2×1 1.2500 0.7500 | E2 = 2×1 0.7500 1.2500 |
| --- | --- |

This agrees with the answer of method 1.

Method 3: Jordan form

The eigenvalue of this matrix is

A-λI=-λ11-λ

detA-λI=λ2-1=λ+1λ-1

λ=±1  .

The eigenvectors become,

v1=11,  λ1=1

v2=-11,  λ2=-1

∴T=1-111

The transition matrix becomes

Φt=Teλ1t00eλ2tT-1

Φt=1-111et00e-t1211-11

In MATLAB, if t=ln2

% Method 2 numerical simulation

[v, d] = eig(A)

T = [-1, 1; 1, 1];

Phi = T*[exp(d(1,1)*t), 0; 0, exp(d(2,2)*t)]/T

Phi = 2×2 1.2500 0.7500 0.7500 1.2500

Which is equal to eAt.

Method 4: Laplace style

We already know the eigenvalues and eigenvectors, so we get the following relationship

sI-A-1=ss+1s-1s-1s+1s-1s-1s+1s-1ss+1s-1=ss+1s-11s+11s+1ss+1s-1

Since,

ss+1s-1=121s+1+1s-1

eAt=L-1sI-A-1

eAt=12e-t+ete-te-t12e-t+et

Plug in t=ln2,

eAt=1.250.750.751.25  .

Exercise 3

Compute eAt for matrix

A=3-5-53

- Using the eigenvalues and eigenvectors of A;

- Using the Laplace Transform

(a).

The eigenvalue of this matrix is

A-λI=3-λ-5-53-λ

detA-λI=3-λ2-25=8-λ-2-λ

λ=-2, 8  .

The eigenvectors become,

v1=11,  λ1=-2

v2=-11,  λ2=8

∴T=1-111

The transition matrix becomes

eAt=Teλ1t00eλ2tT-1

eAt=111-1e-2t00e8t12111-1

eAt=0.5e-2 t+0.5e8 t0.5e-2 t-0.5e8 t0.5e-2 t-0.5e8 t0.5e-2 t+0.5e8 t  .

(b).

Since we already know the eigenvalues and eigenvectors

sI-A-1=s-3s+2s-85s+2s-85s+2s-8s-3s+2s-8

Since,

s-3s+2s-8=0.5s-8+0.5s+2

5s+2s-8=0.5s-8-0.5s+2

the answer becomes

eAt=L-1sI-A-1

eAt=0.5e-2 t+0.5e8 t0.5e-2 t-0.5e8 t0.5e-2 t-0.5e8 t0.5e-2 t+0.5e8 t  .

Exercise 4

Compute eAt at t=ln2 for

A=01-10

Using MATLAB, we can compute the results

A = [0, 1; -1, 0];

t = log(2);

eA = expm(A*t)

eA = 2×2 0.7692 0.6390 -0.6390 0.7692

Exercise 5

Obtain (by hand) the state response x∙ of each of the following systems due to a unit impulse input and the zero initial conditions. For each case, determine whether the response contains all the system modes.

- .

x1=-5x1+2x2+ux2=-12x1+5x2+u

(b).

x1=-5x1+2x2+ux2=-12x1+5x2+2u

(c).

x1=-5x1+2x2+ux2=-12x1+5x2+3u

The A matrix for all three (a)~(c) are the same,

A=-52-125

But the B matrix is different. If the input is a unit impulse input ut=δt, and we know that function of xt becomes

xt=eAtx0+eAtB  .

Since we assume a zero initial condition

xt=eAtB  .

We know can compute eAt.

The eigenvalue of this matrix is

A-λI=-5-λ2-125-λ

detA-λI=(-5-λ)(5-λ)+24=1-λ-1-λ

λ=±1  .

The eigenvectors become,

-62-124v1=00

v1=1/31,  λ1=1

And,

-42-126v2=00

v2=1/21,  λ2=-1

∴T=1132

The transition matrix becomes

eAt=Teλ1t00eλ2tT-1

eAt=1132e-2t00e8t-12-1-11

eAt=3e- t-2e tet-e- t6e- t-6e t3e t-2e- t  .

(a).

For this given system

B=11

Hence,

xt=eAtB

xt=3e- t-2e tet-e- t6e- t-6e t3e t-2e- t11

xt=2e- t-e t4e- t-3e t  .

(b).

For this given system

B=12

Hence,

xt=eAtB

xt=3e- t-2e tet-e- t6e- t-6e t3e t-2e- t12

xt=e- t2e- t

(c).

For this given system

B=13

Hence,

xt=eAtB

xt=3e- t-2e tet-e- t6e- t-6e t3e t-2e- t13

xt=e t3e t

We have a mode of continuously growing do not have the other two modes. So we cannot observe all modes with these responses.

Exercise 6

Consider the system with input u output y and state variables x1,∙∙∙,x4 described by

x1=-x1x2=x1-2x2x3=x1-3x3+ux4=x1+x2+x3+x4y=x3

Obtain (by hand) an expression for the impulse response of this system. Does it contain all the state space modes?

The A matrix is

A=-10001-20010-301111

The state space

x=Ax+Bu

y=Cx+Du

for this system is represented by the matrices

B=0010,  C=0010,  D=0

If the input is a unit impulse input ut=δt, we know that the output response is expressed as

Gt=CeAtB  .

Assuming a zero initial condition for input response

xt=eAtB  .

We know can compute eAt.

The eigenvalue of this matrix is

A-λI=-1-λ0001-2-λ0010-3-λ01111-λ

detA-λI=λ-1λ+1λ+2λ+3

λ=±1, -2, -3  .

The eigenvectors become,

-20001-30010-401110v1=00

v1=0001,  λ1=1

And,

-1000100010-101113v2=00

v2=0-301,  λ2=-2

And,

2000110010101114v3=00

v3=00-41,  λ3=-3

And,

00001-10010-201112v4=00

v4=-0.8-0.8-0.41,  λ4=-1

∴T=000-0.80-30-0.800-4-0.41111

The transition matrix becomes

eAt=Teλ1t0000eλ2t0000eλ3t0000eλ4tT-1

eAt=e- t000e- t-e-2 te-2 t000.5e-t-0.5e-3 t0e-3 t00.3333e-2t-1.25e-t+0.125e-3t+0.7917et0.3333et-0.3333e-2t0.25et-0.25e-3tet  .

Thus,

xt=eAtB

xt=00e-3 t0.25et-0.25e-3t

Gt=CeAtB

Gt=e-3t

And, the output response is characterized as

yt=Cet-t0x0+t0tGt-τuτdτ

x0=0

Since the input is an impulse input, yt=Gt

yt=e-3t

We have only a continuously decaying but do not have the other two. So we cannot observe all modes with these responses.

Exercise 7

Consider an LTI system described by

x1=x2x2=-2x1-3x2+uy=3x1-x2

Is there a persistent input (does not go to zero) u for which the corresponding output always going to zero regardless of initial conditions? If answer is yes provided an example.

The A matrix is

A=01-2-3

The state space

x=Ax+Bu

y=Cx+Du

for this system is represented by the matrices

B=01,  C=3-1,  D=0

We can compute the eigenvalues and eigenvectors as well as eAt using MATLAB

λ= -1, -2

v=1-1,-12

T=1-1-12

eAt=2e-t-e-2te-t-e-2t2e-2t-2e-t2e-2t-e-t

Consider the term

CeAt-t0xt0-xpt0

=CTeΛt-t0T-1xt0-xpt0

=CTe-t-t000e-2t-t0T-1xt0-xpt0

When t →∞, this goes to 0. Thus, A is stable.

The steady state part of the output response is defined as

ysst=Gλeλtv.

If Λ is 0 of GΛ, then the steady state response is also 0.

Next we find,

Gs=CsI-A-1B+D=3-ss2+3s+2

This goes to zero when s=3. Thus, λ= 3

If the input of this system is

ut=e3tv

With this input the steady state part of the output response becomes 0

We verify this with Simulink

% Matrices

A = [0, 1; -2, -3];

B = [0; 1];

C = [3, -1];

D = 0;

lambda = 3;

% random numbers between 0 and 20

low = -20; high = 20;

x01 = low + (high - low) * rand(2,1)

x02 = low + (high - low) * rand(2,1)

sim('ex7_v2');

This is the exponential input

The responses were (for 2 random initial conditions)

Exercise 8

Consider an LTI system described by

x1=x2x2=-2x1-3x2+uy=-x1-3x2+u

Is there a persistent input (does not go to zero) u for which the corresponding output always going to zero regardless of initial conditions? If answer is yes provided an example.

The A matrix is

A=01-2-3

The state space

x=Ax+Bu

y=Cx+Du

for this system is represented by the matrices

B=01,  C=-1-3,  D=1

We can compute the eigenvalues and eigenvectors as well as eAt using MATLAB

λ= -1, -2

v=1-1,-12

eAt=2e-t-e-2te-t-e-2t2e-2t-2e-t2e-2t-e-t

Consider the term

CeAt-t0xt0-xpt0

=CTeΛt-t0T-1xt0-xpt0

=CTe-t-t000e-2t-t0T-1xt0-xpt0

When t →∞, this goes to 0. Thus, A is stable.

The steady state part of the output response is defined as

ysst=Gλeλtv.

If Λ is 0 of GΛ, then the steady state response is also 0.

Next we find,

Gs=CsI-A-1B+D=s2+1s2+3s+2

This goes to zero when s=±j. Thus, λ= ±j

If the input of this system is

ut=cost

With this input the steady state part of the output response becomes 0

Verify this with Simulink

% Matrices

A = [0, 1; -2, -3];

B = [0; 1];

C = [-1, -3];

D = 1;

lambda = 1;

% random numbers between -20 and 20

low = -20; high = 20;

x01 = low + (high - low) * rand(2,1)

x02 = low + (high - low) * rand(2,1)

sim('ex7_v2');

This is the cosine input

The responses were this (for 2 random inputs)

Exercise 9

Disturbing the cart. Consider the pendulum cart system with parameter set P4.

We will subject it to passive stabilization and a disturbance input w, that is, we let

u=-ky-cy+w

where k>0 and c>0. Regard the resulting system as an input-output system with input w and output y and answer the following questions.

- Using MATLAB, obtain the poles and zeros of the system linearized about E1.

- Consider a sinusoidal disturbance input of the form

wt=asinωt

Choose ω so that the steady state response of the linearized system to this disturbance is zero. Simulate both the nonlinear system and the linearized system with zero initial conditions and this disturbance.

|  | m0 | m1 | m2 | l1 | l2 | g | u |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 2 | 1 | 1 | 1 | 1 | 1 | 0 |
| P2 | 2 | 1 | 1 | 1 | 0.99 | 1 | 0 |
| P3 | 2 | 1 | 0.5 | 1 | 1 | 1 | 0 |
| P4 | 2 | 1 | 1 | 1 | 0.5 | 1 | 0 |

E1:      ye,θ1e,θ2e=0,0,0

(a).

The linear Simulink model is the following

The “inputSystem” subsystem is the following

The Embedded MATLAB Blocks are the following

Function:

function y = fcn(u, data, data1, data2)

%{

EMBEDDED MATLAB BLOCK FUNCTION

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -m1*l1*sin(u(1))*u(3)*u(3) - m2*l2*sin(u(2))*u(4)*u(4)...

- m1*g*sin(u(1))*cos(u(1)) - m2*g*sin(u(2))*cos(u(2))...

+ u(5);

den = m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2;

y = num / den;

end

Function1:

function y = fcn(u, data, data1, data2)

%{

EMBEDDED MATLAB BLOCK FUNCTION1

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -(m1*l1*cos(u(1))*sin(u(1))*u(3)*u(3) + m2*l2*cos(u(1))*sin(u(2))*u(4)*u(4))...

+ m2*g*(sin(u(1))*cos(u(2))^2 - cos(u(1))*sin(u(2))*cos(u(2)))...

- (m0 + m1 + m2)*g*sin(u(1)) + u(5)*cos(u(1));

den = l1*(m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2);

y = num / den;

end

Function2:

function y = fcn(u, data, data2, data1)

%{

EMBEDDED MATLAB BLOCK FUNCTION2

%}

m0 = data(1); m1 = data(2); m2 = data(3); l1 = data1(1); l2 = data1(2);

g = data2;

num = -(m1*l1*cos(u(2,1))*sin(u(1))*u(3)*u(3) + m2*l2*cos(u(2))*sin(u(2))*u(4)*u(4))...

+ m1*g*(sin(u(2))*cos(u(1))^2 - cos(u(2))*sin(u(1))*cos(u(1)))...

- (m0 + m1 + m2)*g*sin(u(2)) + u(5)*cos(u(2));

den = l2*(m0 + m1 + m2 - m1*cos(u(1))^2 - m2*cos(u(2))^2);

y = num / den;

end

For the equilibriums conditions E1, we set the initial conditions of the integrator block of y, θ1, and θ2 correspondingly to ye, θ1e, θ2e; like in the following windows,

And set arbitary values for the constants k and c (which presumably are the spring constant and damping coefficient)

| k | c |
| --- | --- |
| 1 | 1 |

Then we run the following MATLAB code to find the trim conditions and perform linearization on the system.

% Set parameters and equilibrium conditions

m = [2,1,1]; l = [1,0.5]; g = 1; % P4

ye = 0; theta1e = 0; theta2e = 0; % E1

k = 1;    % spring constant

c = 1;    % damping coefficient

xe = trim("db_pend_cart_lin_modInput")

[A, B, C, D] = linmod('db_pend_cart_lin_modInput',xe)

sys_ss = ss(A, B, C, D);  % get the state space system

sys_tf = tf(sys_ss)  % get the transfer function

p = pole(sys_tf)  % get the eigenvalues

% Define a transfer function for each one to obtain the zeros

tf1 = tf(sys_tf.Numerator(1), sys_tf.Denominator(1))

tf2 = tf(sys_tf.Numerator(2), sys_tf.Denominator(2))

tf3 = tf(sys_tf.Numerator(3), sys_tf.Denominator(3))

z1 = zero(tf1)

z2 = zero(tf2)

z3 = zero(tf3)

p1 = pole(tf1)

p2 = pole(tf2)

p3 = pole(tf3)

The results are the following

| A = 6×6 0 0 0 1.0000 0 0 0 0 0 0 1.0000 0 0 0 0 0 0 1.0000 -0.5000 -0.5000 -0.5000 -0.5000 0 0 -0.5000 -1.5000 -0.5000 -0.5000 0 0 -1.0000 -1.0000 -3.0000 -1.0000 0 0 | B = 6×1 0 0 0 0.5000 0.5000 1.0000 |
| --- | --- |
| C = 3×6 1 0 0 0 0 0 0 1 0 0 0 0 0 0 1 0 0 0 | D = 3×1 0 0 0 |

The transfer function for the first input “tf1” (for y):

tf1 = 0.5 s^4 + 6.688e-17 s^3 + 1.5 s^2 + 6.688e-17 s + 1 --------------------------------------------------- s^6 + 0.5 s^5 + 5 s^4 + 1.5 s^3 + 5.5 s^2 + s + 1 Continuous-time transfer function.

The corresponding poles and zeros

| p1 = 6×1 complex -0.1259 + 1.8518i -0.1259 - 1.8518i -0.0205 + 1.1189i -0.0205 - 1.1189i -0.1036 + 0.4702i -0.1036 - 0.4702i | z1 = 4×1 complex -0.0000 + 1.4142i -0.0000 - 1.4142i 0.0000 + 1.0000i 0.0000 - 1.0000i |
| --- | --- |

The transfer function for the first input “tf2” (for θ1):

tf2 = 0.5 s^4 + 8.668e-17 s^3 + s^2 + 1.742e-16 s + 4.697e-18 ------------------------------------------------------- s^6 + 0.5 s^5 + 5 s^4 + 1.5 s^3 + 5.5 s^2 + s + 1 Continuous-time transfer function.

The corresponding poles and zeros

| p2 = 6×1 complex -0.1259 + 1.8518i -0.1259 - 1.8518i -0.0205 + 1.1189i -0.0205 - 1.1189i -0.1036 + 0.4702i -0.1036 - 0.4702i | z2 = 4×1 complex 0.0000 + 1.4142i 0.0000 - 1.4142i -0.0000 + 0.0000i -0.0000 - 0.0000i |
| --- | --- |

The transfer function for the first input “tf3” (for θ2):

tf3 = s^4 + 2.388e-16 s^3 + s^2 + 1.254e-16 s - 6.89e-18 -------------------------------------------------- s^6 + 0.5 s^5 + 5 s^4 + 1.5 s^3 + 5.5 s^2 + s + 1 Continuous-time transfer function.

The corresponding poles and zeros

| p3 = 6×1 complex -0.1259 + 1.8518i -0.1259 - 1.8518i -0.0205 + 1.1189i -0.0205 - 1.1189i -0.1036 + 0.4702i -0.1036 - 0.4702i | z3 = 4×1 complex -0.0000 + 1.0000i -0.0000 - 1.0000i -0.0000 + 0.0000i 0.0000 + 0.0000i |
| --- | --- |

(b).

Since the code in part (a)

[xe, ue, ye, dxe] = trim("db_pend_cart_lin_modInput")

where the system was linearized for an equilibrium point of (0, 0, 0). The equilibrium condition for the input w was found to be 0 by the variable “ue” which is highlighted red above.

Thus, if w=0

wt=asinωt=0

and

ω= 0 or π

For the initial condition,

y=ye+δyθ1=θ1e+δθ1θ2=θ2e+δθ2

Thus,

yi=ye+δyiθ1i=θ1e+δθ1iθ2i=θ2e+δθ2i

For E1, the initial condition ye,θ1e,θ2e=0,0,0

δyi,δθ1i,δθ2i=0,0,0=0,0, 0

Then,

(yi,θ1i , θ2i) = 0,0, 0

For the nonlinear system we use the following model

The subsystem “inputSystem” is the following

The Embedded MATLAB Blocks are all the same as the linearization model.

For the linearized one we use the following model

Using MATLAB, we can solve Gs of this matrix A for the linearized model it becomes

Gs=CsI-A-1B+D

=1.6266s4+4.8678s2+3.24533.2452s6+1.6226s5+1.6226s4+4.8678s3+17.849s2+3.2452s+3.24521.6226s4+3.2452s23.2452s6+1.6226s5+1.6226s4+4.8678s3+17.849s2+3.2452s+3.24523.2452s4+3.2452s23.2452s6+1.6226s5+1.6226s4+4.8678s3+17.849s2+3.2452s+3.2452

Solving each numerator to be zero, we find that the combination of (for the input of y) which corresponds to the first row

s=-1.4142j1.4142jj-j

So, for when s=j  we can choose an omega of 1!

ω= 1

Then we have the following plot of the simulation.

The y-displacement does decay to 0. So the simulation is congruent with our results.

This was done with the following code

% (b)

warning("off")

% Set parameters and equilibrium conditions

m = [2,1,1]; l = [1,0.5]; g = 1; % P4

ye = 0; theta1e = 0; theta2e = 0; % E1

k = 1;    % spring constant

c = 1;    % damping coefficient

a = 1;   % amplitude in disturbance

omega = 1;

xe = trim("db_pend_cart_lin_modInput")

[A, B, C, D] = linmod("db_pend_cart_lin_modInput",xe)

% % % Proof

% syms t s

% G = vpa(simplify(C*inv(s*eye(6)-A)*B+D))

% res1 = solve(G(1)==0, s)

% res2 = solve(G(2)==0, s)

% res3 = solve(G(3)==0, s)

% Initial conditions for the perturbation variables

del_yi = 0; del_theta1i = 0; del_theta2i = 0;

del_yi_dot = 0; del_theta1i_dot = 0; del_theta2i_dot = 0;

% Initial conditions for the variables

yi = ye + del_yi;

theta1i = theta1e + del_theta1i;

theta2i = theta2e + del_theta2i;

yi_dot = del_yi_dot;

theta1i_dot = del_theta1i_dot;

theta2i_dot = del_theta2i_dot;

% Linearized system

IC_lin = [del_yi, del_theta1i, del_theta2i, 0, 0, 0];

lin_res = sim('ss_lin_sys');

% Nonlinear system

nonlin_res = sim('db_pend_cart_nonlin_modInput')

% get values from results

tspan_lin  = lin_res.tout;

y_lin      = lin_res.lin_del_y.signals.values;

theta1_lin = lin_res.lin_del_theta1.signals.values;

theta2_lin = lin_res.lin_del_theta2.signals.values;

tspan_nonlin  = nonlin_res.tout;

y_nonlin      = nonlin_res.y_nonlin.signals.values;

theta1_nonlin = nonlin_res.theta1_nonlin.signals.values;

theta2_nonlin = nonlin_res.theta2_nonlin.signals.values;

% Plotting

fig1 = figure('Renderer',"painters", 'Position', [10 10 900 1000]);

subplot(3,1,1)

hold on; grid on; grid minor; box on;

plot(tspan_lin, y_lin)

plot(tspan_nonlin, y_nonlin)

ylabel('y [m]')

hold off

subplot(3,1,2)

hold on; grid on; grid minor; box on;

plot(tspan_lin, theta1_lin)

plot(tspan_nonlin, theta1_nonlin)

ylabel('$\theta_1$ [rad]')

hold off

subplot(3,1,3)

hold on; grid on; grid minor; box on;

plot(tspan_lin, theta2_lin)

plot(tspan_nonlin, theta2_nonlin)

ylabel('$\theta_2$ [rad]')

xlabel('time, [sec]')

h = legend('linear','nonlinear'); set(h, 'Position',  [0.8, 0.05, .1, .025]);

hold off

sgtitle({['P4/E1 Time History of Double Pendulum Cart System with Passive ' ...

'Stabilization'], ' and Disturbance Input $0\leq t \leq 100$ - T. Koike'})

saveas(fig1, 'p9_b.png')